Petrofac Engineering & Production Services Asset Management WPI NZ DATA CLEANING GUIDE PAMEUR-999999001-XXX-XXX-XXX-XXX

| Approver: |  |  |  |
| --- | --- | --- | --- |
| Job Title: |  |  |  |
| Owner: |  |  |  |
| Job Title: |  |  |  |
| Released: |  | Reviewed: |  |

DISTRIBUTION LISTING

| Copy Number |  | Document Holder |  |  |
| --- | --- | --- | --- | --- |
| Master (electronic) |  | Project Sharepoint Site |  |  |
| Controlled Hardcopy Distribution for those without BMS Access |  |  |  |  |
| No. | Name |  | Job Title | Full Postal Address |
| 001 | NONE |  |  |  |

REVISION CONTROL

| Rev No. | Description of Change | Owner | Date |
| --- | --- | --- | --- |
| 01 | Added Section 11. Application Updated section 23.2 |  | 13 Oct 21 |
| 02 | Supplier names added 9.6 |  | 15 Oct 21 |
| 03 |  |  |  |

Document applicability

Applicable Area

ASSOCIATED DOCUMENTS

| Doc No. | Document Title |
| --- | --- |

© Petrofac Facilities Management Limited 2017

Copyright in the whole and every part of this document belongs to Petrofac Facilities Management Limited. This document is confidential to Petrofac Facilities Management Limited and its associated Petrofac companies. The information herein contained may not be used, sold, transferred, disclosed, copied or reproduced in whole or part in any manner, form or media to any person without the prior written consent of Petrofac Facilities Management Limited.

Table of CONTENTS

1.0	Purpose & Scope	7

1.1.	Purpose	7

1.2.	Scope	7

1.3.	Document Management	7

1.4.	Software & Technical Dictionary	7

2.0	Overview	8

2.1.	Physical Facilities	8

2.2.	Units of Measure and Issue	8

2.3.	Material Groups	8

3.0	Cleaning Levels	9

3.1.	Cleaning Level	9

3.2.	Level 1: Classify and Match	9

3.3.	Level 2: Normalisation and Attribute Value Extraction	9

3.4.	Level 3: Enhance and Research	9

3.5.	Level 3: Commodity materials	10

3.6.	Level 3 production reference	10

3.7.	Level 4: Walk-down	10

3.8.	Masterpiece	10

4.0	Project Workflow	11

4.1.	Masterpiece workflow	11

4.2.	Pending	11

4.3.	Rework	11

5.0	Dictionary	11

5.1.	Nouns, Modifiers, Attribute labels	11

5.2.	Unapproved Attribute Values	11

6.0	Language	12

7.0	Part Numbers	12

7.1.	Blank part numbers	12

7.2.	Deletions	12

7.3.	Additional part numbers	12

7.4.	Replacements/supersessions	12

7.5.	Length	12

7.6.	Items without unique part numbers	12

8.0	Model Numbers	13

8.1.	Model/Figure numbers vs Part-numbers	13

8.2.	Notes	13

9.0	Manufacturers	13

9.1.	Blank Manufacturers	13

9.2.	Deletions	13

9.3.	Additional manufacturers	13

9.4.	New manufacturers	13

9.5.	Manufacturers vs Suppliers	13

9.6.	Supplier Names	14

10.0	Switch board for the C-Screen	14

11.0	Application (ATTRIBUTE)	14

12.0	Legacy Data Information	15

12.1.	Series	15

12.2.	Tag Number	15

12.3.	Serial Number	15

12.4.	EPC Number	15

12.5.	Legacy Number	15

12.6.	OLD Part number	15

12.7.	Order Number	15

12.8.	Lot Number	16

12.9.	Job Number	16

12.10.	Stock Number	16

12.11.	Rebuild	16

12.12.	Service	16

12.13.	Maintenance	16

12.14.	Overhaul	16

13.0	Modifier “ANY”	17

13.1.	Allowable	17

14.0	Type	17

14.1.	Function	17

14.2.	Generic	17

14.3.	Manufacturer-specific information	17

15.0	Surface Treatment	17

16.0	Connection	17

17.0	Pressure Rating	17

17.1.	LB versus psi (Bar/PSI/kpa)	17

18.0	Size	18

18.1.	Conversions (metric and imperial)	18

18.2.	Bearings	18

18.3.	Oil seals	18

18.4.	Belts	18

18.5.	Filters	18

18.6.	Fraction format	19

18.7.	Conversions (fractions and decimals)	19

18.8.	Package size	19

18.9.	Repetitive sizes	19

18.10.	Reducing sizes	19

18.11.	Trailing zeroes	19

18.12.	Multiple values	19

18.13.	DASH size	19

19.0	Range	20

19.1.	Low to High	20

20.0	Construction Materials	20

20.1.	Types and Grades	20

21.0	Attribute Values	20

21.1.	Use existing values	20

21.2.	Synonyms	20

21.3.	Punctuation	20

21.4.	Separator	20

21.5.	Multiple values	21

21.6.	Plurals	21

21.7.	Ratios	21

21.8.	With	21

21.9.	Kits	21

22.0	Units of Measure (UoM)	21

22.1.	Attribute	21

22.2.	Base	21

22.3.	Volumetric	21

23.0	Notes	21

23.1.	Notes Fields	22

23.2.	Equipment notes (i.e. Application)	22

23.3.	Purchasing notes	22

23.4.	General notes	22

23.5.	Completing the Notes Fields	22

24.0	Noun-Modifier specific rules	22

24.1.	Angle, Structural	22

24.2.	Assembly	22

24.3.	Bearing Cup and Cones	22

24.4.	Bolts	23

24.5.	Elbow	23

24.6.	Cap, Pipe	23

24.7.	Chain, Drive and Link, Chain	23

24.8.	Cylinders	23

24.9.	Fasteners	23

24.10.	Flat Steel	23

24.11.	Gas Springs	23

24.12.	All Gaskets	24

24.13.	Gauges	24

24.14.	Grating, Any	24

24.15.	Heater Elements	24

24.16.	Kits	24

24.17.	Lug, Compression	24

24.18.	Malleable iron	24

24.19.	O-Rings	24

24.20.	PCBs and Modules	24

24.21.	Photoelectric and Proximity Switches	24

24.22.	Pins	25

24.23.	Plate	25

24.24.	Sets	25

24.25.	Shackle	25

24.26.	Sheetmetal	25

24.27.	Standard	25

24.28.	Stud	25

24.29.	Tube	25

24.30.	Tubing, Structural	25

25.0	Description Settings:	26

25.1.	Short Description:	26

25.2.	Long Description:	27

APPENDIX 1: xxxx	28

TABLE OF TABLES

Table 1 – yyyy	25

List of Figures

Figure 1 - Masterpiece QA Status Process	11

Purpose & Scope

Purpose

The purpose of the Data Cleaning Guide (DCG) is to describe the rules mandated for completing Item Master data cleaning for the WPI NZ Project.

Scope

This DCG applies to all WPI NZ cataloguing requirements. All Maximo material types must comply with this guideline.

Document Management

This document is a sub set of the core WPI NZ Item Masters standards. This DCG will be maintained by Petrofac during the course of the Data Cleaning project. Petrofac will hand over ownership of the standard to WPI NZ upon completion of the Data Cleaning project.

Software & Technical Dictionary

The MasterPiece Software & Technical Dictionary used for the WPI NZ project is accessed via a licence.  WPI NZ will not have access to the MasterPiece Software & Technical Dictionary after the data cleaning is complete.

Should WPI NZ require access to a licence Petrofac will provide a variation after the data cleaning is complete.

Overview

Physical Facilities

This project scope is to clean Item Master lines at the Master Maximo level.

Units of Measure and Issue

Catalogue items will be assigned a “Base” Unit of Measure being maintained in Maximo.  International Standards Organisation (ISO) Standards will be cross referenced. Consideration shall be given to the distinction between the catalogued Base Unit of Measure and the Order Unit (i.e. Unit of Purchase) that may differ and will require a conversion factor to be determined.

Material Groups

Catalogue items will be assigned a Material Group based on the United Nations Standard Products and Services Code (UNSPSC©). Maximo Material Groups will be consistent with this international coding convention but descriptions may differ.

The UNSPSC is an 8-digit code; composed of four parts (each part has two digits). The codes are hierarchical, similar to an outline. As you get deeper in the outline, there is more detail. The most detail is in at the Commodity level. The four parts are:

1.            Segments (general)

2.            Family

3.            Class

4.            Commodity

Example: Segment:

40000000 – Distribution and conditioning systems and equipment and components

Family:

40150000 – Industrial pumps and compressors

Class:

40151500 – Pumps

Commodity:

40151503 – Centrifugal pumps

Cleaning Levels

Cleaning Level

This project will have a blended cleaning level approach. This means that the default cleaning will be dependent on the quality of data provided and if the data cleaning team can enhance this data.

The following describes the range of possible cleaning levels that could be specified if required.

Level 1: Classify and Match

Extract noun-modifier pairs;

Map noun-modifier pairs to a global classification standard and/or Customer classification;

Map base units of measure to a common standard;

Extract and standardize manufacturer names into a separate manufacturer name field;

Extract manufacturer’s part number into a separate manufacturer part number field;

Generate short and long item master descriptions, per the defined rules;

Identify suspected duplicates based on manufacturer name and part number;

Word analysis and dictionary enhancement;

Review results via Quality Assurance process.

Level 2: Normalisation and Attribute Value Extraction

Level 1 above, plus:

Extract available attribute values into appropriate attribute label fields, with their related units of measure;

Review new attribute values to ensure naming conventions and units of measure are correct and consistent. New attribute values are approved, corrected or merged into previously existing values;

Data outside standard attribute fields is added to a notes field;

Identify functional duplicates based on noun, modifier and attribute values.

Level 3: Enhance and Research

Level 2 above, plus:

Use information provided by the manufacturer and supplier name and part numbers to conduct web-based and/or catalogue research to investigate items and:

Identify ambiguous or missing noun-modifiers,

Identify ambiguous or missing attribute values,

Resolve suspected duplicates,

Attempt to complete missing attribute information.

Update manufacturer name and part number

Level 3: Commodity materials

For commodity materials such as Stud Bolts, Gaskets, Piping & Pipe Fittings the manufacturer & part number should not be included where there is sufficient information to build a full technical description.

Level 3 production reference

All level 3 research lines require a reference (web or otherwise) to confirm where the additional information was found. Please indicate where it can be found for example the page number of a PDF or Catalogue and the URL of a webpage.

If two URL’s are considered necessary, add the second one to research notes. If no reference is found, indicate this by adding “No Further Information” in the research panel.

For multi-page references the Page Number should be indicated in Research Notes

Level 4: Walk-down

This involves the physical inspection of the item to determine manufacturer and technical information.  This is outside of the scope of the WPI NZ project

Masterpiece

The level 3 reference in Masterpiece is to be recorded in the Web Address and/or Research Notes field which is visible in the c-screen or accessible via the research button on the e-screen.

Project Workflow

Masterpiece workflow

Figure 1 - Masterpiece QA Status Process

The Data Cleaning Production team will take the line from “Scanned” or “In Progress” to “Checked”.

Data Cleaning Production QC takes line from “Checked” to “Quality Controlled”.

Petrofac QA takes line from “Quality Controlled” to “Quality Approved”.

WPI Cataloguer to take the line from “Quality Approved” to “Accepted”.

Pending

If the original data has conflicting information or cannot be understood, the line will be sent to Pending via the C screen or Mass Assign for further clarification or more information. This is known as raising an issue.

In both cases, when the Pending status is selected a box will appear which must be completed with details of the problem.

This will raise an email to Data Cleaning QA who will escalate the issue to WPI NZ via the agreed escalation process.

Rework

If a line is rejected though the WPI NZ Data Cleaning project QA process, it will be sent to rework through the Masterpiece tool.

Dictionary

Nouns, Modifiers, Attribute labels

If cleaners require new nouns, modifiers or attribute labels, an email should be sent to Petrofac including the noun, modifier, or attribute label required, including a sample line.

Unapproved Attribute Values

New attribute values added to the dictionary will need to be reviewed and either approved or rejected by Petrofac.

Language

All data to be cleansed in UK English.

There is no requirement for translation a foreign language.

Part Numbers

Blank part numbers

Where the part number field is blank and a part number is visible in the short and/or long description, this part number is to be inserted into the part number

field.

Deletions

Where the part number appears to be clearly wrong and no correct part number can be found, set the part number field to blank.

Additional part numbers

If the part number field is populated and an additional part number is provided in the short and/or long description, the cleaner will attempt to verify the correct

part number and insert the other number in the Purchasing notes field.

Replacements/supersessions

Where there is reference to “replaced by” or “superseded by”, the existing old original part number will be used and the new part number captured in Purchasing Notes with the preface of "Superceded with".

To be shown in Long Description

Length

Manufacturer part-numbers should not be less than 3 characters in length.

Items without unique part numbers

Where the parts are known to have different dimensions but traditionally have the same part number, add an extension to the part number to make it unique, using the format – XL, 2, etc.

The parts below are likely to fall into this category:

PPE - shoes, boots, clothing

Valves and pumps that use figure or models as part numbers

Taper lock bushings (SK x 1-1/2)

Sprockets with bores

Pulleys with bores

Drive couplings that use trade size as part number will need bore size

If in doubt about how to adjust the part numbers, ask for confirmation.

Model Numbers

Model/Figure numbers vs Part-numbers

Do not enter model numbers or figure numbers in the part number field unless it acts as the part number.

Examples of items that use Model numbers or Figure numbers as part numbers are Valves and Pumps

These should be entered in part number field with the size, if necessary to prevent incorrect matching

Notes

Enter model numbers in the Purchasing Notes field, with the prefix “Model no:”.

Manufacturers

Blank Manufacturers

Where the manufacturer field is blank and a manufacturer is visible in the short and/or long description, this manufacturer is to be inserted into the

manufacturer field.

Deletions

Where the manufacturer appears to be clearly wrong and no correct manufacturer can be found, set the manufacturer field to blank.

Additional manufacturers

If the manufacturer field is populated and an additional manufacturer is provided in the short and/or long description, the cleaner will attempt to verify the correct

manufacturer and insert the other name in the notes field.

New manufacturers

Where a new manufacturer is required an email should be sent to Nick Waters including the Manufacturer Name required, material number, description and

URL.

Manufacturers vs Suppliers

Ensure that all part numbers and / or names in the manufacturer name and part number fields relate to manufacturers and not suppliers. If they are supplier

details, remove them from the manufacturer fields and record in Purchasing Notes.

Supplier Names

- Corys Electrical	https://corys.co.nz/

- Cleanline		https://www.cleanlinetasman.co.nz/

- Tradezone		https://tradezone.co.nz/

- Blackwoods		https://nzsafetyblackwoods.co.nz/en/

Switch board for the C-Screen

|  | Name | Description | Enabled |
| --- | --- | --- | --- |
| 10.1 | Colloquial | This appears on the bottom left of the C-Screen allowing relevant information to be added to the line. |  |
| 10.1 | Drawing Number | This is on the bottom right of the C-Screen allowing the parts Drawing Number to be added. | YES |
| 10.1 | Show VPI Search Button | This is on the top left of the C-Screen giving you access to VPI. |  |
| 10.1 | Supplier | Supplier information contained in the short or long description fields is to be extracted and placed in Purchasing Notes. Any supplier information loaded to the supplier fields is for reference only. This appears in the middle of the C-Screen where the supplier information for this item can be added. | YES |
| 10.1 | Model Number | This field appears on the bottom left of the C-Screen where you may add the Model Number. | YES |
| 10.1 | Trade Name | This field appears on the bottom left of the C-Screen where you may add the Trade Name. |  |
| 10.1 | Enable Related Docs | This appears on the bottom right of the C-Screen where documents relevant to the item can be attached. |  |
| 10.1 | Enable Research | You can add additional research information on the bottom right side of the C-Screen. | YES |

Application (ATTRIBUTE)

The WPI Material descriptions refer to various equipment identifiers/’Tags’.

Other examples include:

- Equipment       HPP = High Pressure Press

- Equipment       BFB1 = Bale Forming Press 1

- Equipment       BFB2 = Bale Forming Press 2

- Equipment       BPP1 = Bale Packing Press 1

- Equipment       BPP2 = Bale Packing Press 2

- Equipment       P### = Pump tag

- Equipment       T### = Screw Conveyor

- Equipment       A# = Andritz

- Attribute          OA = Open Area (screen)

This information should be added to the ‘Application’ Attribute in the template.

Legacy Data Information

Series

Unless otherwise advised put into Purchase Notes

Tag Number

Unless otherwise advised put into Application

Serial Number

Unless otherwise advised put into Purchase Notes

EPC Number

Unless otherwise advised put into Purchase Notes

Legacy Number

Unless otherwise advised put into Purchase Notes

OLD Part number

Unless otherwise advised put into Purchase Notes

Order Number

Unless otherwise advised put into Purchase Notes

Lot Number

Unless otherwise advised put into Purchase Notes

Job Number

Unless otherwise advised put into Purchase Notes

Stock Number

Unless otherwise advised put into Purchase Notes

Rebuild

Unless otherwise advised put into General Notes

Service

Unless otherwise advised put into General Notes

Maintenance

Unless otherwise advised put into General Notes

Overhaul

Unless otherwise advised put into General Notes

Modifier “ANY”

Allowable

Where possible always assign a modifier to an item to avoid Modifier ANY.

There are some exceptions, such as, O-Ring, Boots, Pants. It is ok to have the modifier of ANY in these instances

Type

Function

Type should not be used to describe the function of the item. There are some exceptions, e.g. GASKET, ANY – with a TYPE of Head, Cover etc. When in

doubt, ask for assistance from Petrofac QA.

It also should not be used to describe the functional location (Top, Bottom, Front ) These should be captured in Construction

Generic

Generic terms such as Complete, Standard and Generic should not be used in Type.

Manufacturer-specific information

Type attribute is for descriptive information only and not Manufacturer or Supplier types such as MS109, or Models or Model Numbers.

Surface Treatment

12.1 If the material indicates "Black", such as Black Malleable Iron, enter BLACK in SURFACE TREATMENT

Connection

If the connection is repetitive (e.g. MNPT X MNPT, MALE X MALE) only enter the connection value once (e.g. MNPT, MALE).

Pressure Rating

LB versus psi (Bar/PSI/kpa)

LB should be used as attribute UoM ONLY for Pressure Classes. These are on pipe, pipe fittings, some valves and spiral wound gaskets. All others will be UoM of psi. See table below

It is possible to have a coincidental psi pressure rating that appearsin below table. That does not make it a pressure class. If you have a pressure gauge or some other noun, that does not use pressure classes use psi even if the value is also a pressure class. If you have a connection type of 150lb flange, use lb as a flange is a pipe fitting

Class Rating

[#]

150

300

400

600

900

1500

2500

Size

Conversions (metric and imperial)

Do not convert customer data from metric to imperial or vice versa.

Bearings

Bearings sizes should be carried out to 4 decimals (2.5478) for imperial and 2 decimals (25.15) for metric. Drop all trailing zeros on imperial and metric sizes

(0.6250 = 0.625, 2.0000 = 2)

Oil seals

Oil seals sizes (and only oil seals, not other seals) should be entered in decimal form and carried out to 3 decimals (2.547) for imperial and 2 decimals (25.15) for metric. Drop all trailing zeros on imperial and metric sizes (0.0620 = 0.062, 2.000 = 2).

Belts

Belt sizes for modifiers of timing, v, and conveyor should be entered in decimal form and carried out to 3 decimals (2.547) for imperial and 2 decimals (25.15) for metric. Drop all trailing zeros on imperial and metric sizes (0.620 = 0.62, 2.000 = 2) Other modifiers are not to be converted to decimal when a fractional size is given

Filters

Filter dimensional sizes should be entered in decimal form and carried out to 4 decimals (2.547) for imperial and 2 decimals (25.15) for metric. Drop all trailing zeros on imperial and metric sizes (0.620 = 0.62, 2.000 = 2).  Connection sizes should remain as fractional where applicable

Fraction format

Fractions are to be displayed in the format 1-1/2 and not 1.1/2 or 1 ½

Conversions (fractions and decimals)

In general, excluding the above mentioned, all imperial decimals should be converted to fractions when possible (0.375 = 3/8 or 0.5 = 1/2). Metric sizes should

never be converted to fractions. Where applicable, conversions should be done in accordance with the table below:

Package size

The package size in Master Data should describe the smallest individual package size. How many of these is supplied and in what packaging is described in

Purchasing Notes.

Repetitive sizes

If the pipe or tubing size is repetitive (1/2x1/2) only enter the size value once (1/2).

Reducing sizes

Reducing sizes will be entered large to small (10X8)

Trailing zeroes

Trailing zeroes should be removed from all sizes, regardless of the noun/modifier (1.500 = 1.5)

Multiple values

If there is more than one numeric, they should be written as follows: 3X5 or 1-1/2X3 with no space between X and the numbers.

DASH size

In reference to tubing and hose fittings, some manufacturer will use an AN size in their part numbers. These will identify the fitting sizes. See conversion table below

Conversion to inches table:

| AN size | Tube OD (Hose ID) |
| --- | --- |
| -2 | 1/8” |
| -3 | 3/16” |
| -4 | 1/4” |
| -5 | 5/16” |
| -6 | 3/8” |
| -8 | 1/2” |
| -10 | 5/8” |
| -12 | 3/4” |
| -16 | 1” |
| -20 | 1‐1/4” |
| -24 | 1‐1/2 |
| -28 | 1‐3/4 |
| -32 | 2” |

Range

Low to High

Numbers should be entered low to high and not vice versa.

Construction Materials

Types and Grades

304, 316 and others are a TYPE of stainless steel and not a Grade. They should be entered with the material as ‘304 Stainless Steel’, etc

Attribute Values

Use existing values

When adding new attribute values check for proper spelling and try to mirror formats, styles and values with those that already exist in the list. Before creating a new attribute value always check existing values and ensure that you cannot use a similar existing value. E.g., “pump rear journal” and “pump journal rear” are more than likely the same type item, so use what is there instead of creating a new value.

Synonyms

Sometimes, we may have already caught these instances and created a synonym, in which case if you tried to add new value, it would automatically direct

you to the correct value.

Punctuation

Do not use semi-colons, commas, brackets or the following { }#@~^*!<>&

Separator

If it is necessary to separate values use a forward slash i.e., “/”. Do not leave a space before and after forward slash i.e., black/red not black / red

Multiple values

If there are multiple values in an attribute, enter them alphabetically (carbon steel/rubber instead of rubber/carbon steel).

Plurals

Try not to use plurals in type or other attributes where possible. Adding an “s” where it is not necessary only adds additional values that are neither necessary

nor unique.

Ratios

Ratios should be written in 45:1 format and consideration should be given to whether it is an increasing or reducing ratio so that the numbers are put the right

way around.

With

The word “with” should be abbreviated to “w/ ” (space after /) and “without” to “w/o ” (space after o).

Kits

Add each item separately under the individual attributes CONSISTS OF 1, CONSISTS of 2 etc. For single items do not use a prefix such as 1 X or (1) OF. For

multiple items use the format ‘2 X ORINGS’,

for example.

Units of Measure (UoM)

Attribute

Units of Measure (mm, inch, gm, etc) should be recorded as the Attribute Units of Measure, and not written as part of the Attribute Value.

Base

Where incorrect, Base UoMs are to be changed and standardised according to customer defined list. Agree with customer.

Volumetric

If the Base UoM is volumetric such as Gallon, Quart, Liter, etc, unless the client description specifies otherwise, the Package Size will be 1 Gallon and so on.

Notes

Notes Fields

These are plant specific notes found on the C-screen and the Working screen. They consist of Equipment Notes, Purchasing Notes and General Notes. These

fields are used to store additional information which does not fall into an attribute value or is non-descriptive information

Equipment notes (i.e. Application)

“Where used” or “Application” information does not generally describe an item’s functionality.

Please refer to WPI PDF 95-001 (Drawing Ref) for full Main Equipment illustration – see section 11. Application for usage instruction.

Purchasing notes

This field is for all information related to purchasing the item, including old supplier name and part numbers and delivery information. Do not enter price

information. This includes, item numbers, reference numbers and Model numbers.

General notes

This field is only for item specific, descriptive information that does not fit into the NMQA structure or that are not related to Equipment or Purchasing.

Completing the Notes Fields

Complete the notes fields in a concise and logical style excluding the attribute name: eg. Rather than: "For: valve, Size:2in, Material: carbon steel" use: "for

2in carbon steel valve". For nonspecific

values which are not clear, preface with ‘REF:’.

Noun-Modifier specific rules

Angle, Structural

For attribute Width enter dimensions from big to small i.e. 10x8 not 8x10.

Assembly

An assembly is an assemblage of parts to form one part. If possible, put it to the root noun and use assembly in the attribute Type (Gear Assembly would be

noun Gear and Type Assembly). See also SET and KIT

Bearing Cup and Cones

For Bearing, Cup Bearing, Cone and Bearing, Cup/Cone, do not enter the Type as Roller It is redundant and takes up space in short description

Bolts

If an item is a common Hex or Heavy Hex head bolt, cleanse to Bolt, Machine

For Bolts over 1in with UNC threads AND 8tpi, enter THREAD TYPE as 8UN and do not enter the TPI in THREADS attribute

For Bolts over 1-1/2in with UNF threads AND 12tpi, enter THREAD TYPE as 12UN and do not enter the TPI in THREADS attribute

For Bolts over 1-3/4in with fine threads AND 16tpi, enter THREAD TYPE as 16UN and do not enter the TPI in THREADS attribute

Elbow

a) If an Elbow has a Bend Radius of 1.5D, 3D, 5D, 6D the item will go to Bend, Pipe.

b) If the Elbow has a Bend Radius of 1.5D LONG, the item will go to Elbow, Pipe.

c) If the Bend Radius is less than 1.5 it will go to Elbow.

Cap, Pipe

Connection type should never be male.

Chain, Drive and Link, Chain

Enter the complete Size (Textual) in the part number field, such as 100RIV or 100-2RIV.

Cylinders

a) Air or Air-operated cylinders will go to Cylinder, Pneumatic.

b) If it gives a ‘SQ In’ size or Actuator trade size like ‘200’, it is Actuator Modifier.

c) If it gives a pressure rating but says Hydraulic, it is Cylinder, Hydraulic.

Fasteners

For Bolts, Nuts, Screw and Studs, when it is a metric or a # size, enter in THREAD SIZE as M10, M12, NO 4, NO 10 etc, without a UoM If there is a Head, enter the shape then the drive Example Hex Socket, Flat Slotted

Flat Steel

Goes to Bar, Flat.

Gas Springs

All gas springs are to be cleaned to ABSORBER, SHOCK

All Gaskets

If Garlockor Klinger style is given, enter as Garlock 3000 or other relevant in material attribute The mfr name should be included in the attibute value, such as Garlock 3000

Gauges

If a gauge has a pressure rating and do not also include a vacuum rating, it will go to Gauge, Pressure. If a gauge has a pressure rating AND a vacuum rating, it will go to Gauge, Compound

Grating, Any

The attribute “thickness” refers to the overall thickness of the grating.

Heater Elements

Are retained as Element, Electrical and Type as Heater. The exception is where the input description contains the word “Overload” or the series of the part numbers is of an Overload Heater, in which case it will be classified to Heater, Overload.

Kits

A kit is a grouping of spares parts for repair or maintenance.

These can be different parts and different sizes

See also Assembly and Set

Lug, Compression

Use TIP SIZE attribute for blade and pin type lugs

Fork and Ring type lugs will have a BOLT SIZE

Malleable iron

Fittings made of malleable iron will go to their specific noun with the modifier Pipe.

O-Rings

When enhancing o-rings with a dash number (Reference Standard), use the Actual Size on the o-ring chart NOT the Nominal Size

PCBs and Modules

The number of inputs and outputs will be entered with the FUNCTION Ex 12 Analogue Input, 12 Analogue I/O

Photoelectric and Proximity Switches

All Photoelectric and Proximity Switches are to be cleaned, as applies, to Sensor, Photoelectric and Sensor, Proximity

Pins

Spring and Taper pins go to Pin, Retaining.

Plate

Several are Perforated which will be noted in Type.

Sets

A set is a grouping of the same item with different sizes. Example: Wrench Set

See also Assembly and Kit

Shackle

If “DEE” or “D” is in description, it goes to Shackle, Chain.

Sheetmetal

All gauge plate should be cleansed to Sheetmetal, Any.

Standard

If the term Standard appears alongside a chemical name, clean as Chemical and include Standard in Type as well as the chemical name. Example: Chemical Ammonium Hydrate Standard.

Stud

If they are a threaded connecting stud these will be allocated to Stud, Bolt.

Tube

Any tubes in lengths which appear to be bulk will go to Tubing. Modifiers such as Metallic, Non-Metallic or Any will be noted. A specific cut length of tube, such as 3 in, will go to Tube with appropriate modifier

Tubing, Structural

For attribute Width, there should always be 2 dimensions, even if it is square i.e. 10x10 or 10x8.

Description Settings:

Short Description:

Noun-Modifier: Attributes, Manufacture, Part Number

- No Line breaks

- Abbreviations included in Attributes

- Max of 120 Characters

Example:

--------------------------------------------- CONTINUED---------------------------------------------

Long Description:

Noun-Modifier: Attributes, Manufacture, Part Number

- Line breaks included

- No Attributes

- Max of 120 Characters per line

Example:

# APPENDIX 1: xxxx