Solution Building Block Document

Glencore Copper Global Template

Standardization of

Material Descriptions

Document Meta Data

| Stream Name | Project Name |
| --- | --- |
| Material Management | Glencore Copper Global |

Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 0.1 | 21-9-2018 | Draft | Roelof van den Heuvel |
| 0.2 | 26-9-2018 | Draft | Sabrina Perron-Fortin |
| 0.3 | 26/02/2019 | Final Draft | Roelof van den Heuvel |
| 0.4 | 25/04/2019 | Draft | Carol Nevin |
| 0.5 | 20/08/2019 | Draft | Garth Nevin |
| 0.6 | 23/11/2020 | Draft | Garth Nevin |
| 0.7 | 03/09/2021 | Draft | Garth Nevin |

| Overall Status | Ready for Core Team Review |
| --- | --- |

Contents

Introduction / Purpose	4

Assumptions	4

Basic Rules	4

Solution requirements	5

Short Format Description (SFD) (40 Characters)	5

Original Equipment Manufacturer (OEM) List	5

LONG DESCRIPTION FORMAT	6

1.	Class	6

2.	Reference Text	7

3.	PART NO	7

4.	DRAWING NO	7

5.	CHARACTERISTICS	7

6.	ADDITIONAL FEATURES	7

7.	CLEAN-UP OF ADDITIONAL FEATURES DATA	8

8.	SAP MATERIAL GROUP and VALUATION CLASS	8

9.	MATERIAL TYPE	11

10.	LANGUAGE STANDARD	12

11.	SPECIAL CHARACTERS	12

12.	TOLERANCES	13

13.	SERIES OF NO’S	13

14.	RANGES	13

15.	COLOR	14

16.	DIMENSIONS, SIZES	14

17.	UNIT OF MEASURES/NOTES	14

18.	THREAD ABBREVIATION and NOTES	18

19.	MATERIAL ABBREVIATION	20

20.	GENERAL ABBREVIATION	23

21.	Examples per material (item) types	25

22.	Metric to Imperial Conversion	25

	Sign-off	28

# Introduction / Purpose

The Purpose of this document is so standardise all data (technical) within the value fields, this will ensure that the Master data will be created as a Standard within Glencore. The Standardisation of Noun, Modifier, Characteristics and Values will improve search functions against existing items within the Master data file. In doing so, this will further assist when duplication resolution is to be carried out on items.

This document describes the Metadata that is necessary within Material Management (Master Data to facilitate the process required for the Standardisation of Material descriptions for Glencore Copper Global.)

Cataloging of all Materials items have the following benefits for Master Data.

Glencore will have a single point of entry for all Material Master Items with a standard format for requesting creations/changes/extension in place. No changes are to be made by anyone except the approved cataloguer.

Enables quick and easy identification of all parts (which are manufactured by one or more manufacturers).  Parts having the same form, fit, function and similar technical performance characteristics will be visible.

Enables Glencore to establish standardised, and structured database, this will ensure accurate, consistent information which can be used throughout the Company/Group.

Enables Glencore to identify contractors (OEM, Suppliers and Manufacturers) which can supply materials. The company at the same time can then identify preferred contractors (OEM, Suppliers and Manufacturers) thus enabling Glencore to negotiate to have items placed on contracts or current contracts can be renegotiated for better pricing for procurement purposes.

All end users will be able to see the item description. End users will be able to do searches.

Glencore will have visibility of poor description. Thereby addressing and ensuring a Global standard.

By not duplicating material masters, inventory stock levels can be reduced.

Furthermore, Free Text procurement of items can be stopped.

# Assumptions

- All rules set out in this document shall be used in creation of all new Material descriptions and for the update/changes of old descriptions.

- Exceptions could occur.

# Basic Rules

The follow rules are to be applied

- Always ask yourself “WHAT DO I HAVE IN MY HAND” – what is the Noun?

- Respect the format proposed for every value you want to add.

- Respect the value standardization.

- Space Allowed. Any multi-spacing within the value range is replaced with a single space.

- Correct all spelling mistakes.

- No punctuation and accent.

- Only some special characters are allowed. See p.10 of this document.

- No plural in the Noun.

- No abbreviation in the Noun.

- No Size Conversions are to be done EG 1-1/2in to 38.1mm or visa versa.

- All imperial sizes to be fractionalised – not decimals. I.e. 1-1/2 not 1.5 inches. Note the format that fractions are capture. (https://www.metric-conversions.org is a great site to use.)

- Use existing abbreviations/acronyms only. See pg.12-23 of this document.

- No base unit of measure (BUOM) in the short description.

- All data is to be in upper case.

- Additional Features – Must have direct value add to class.

- Please see below Rules for short description regarding OEM and Non-OEM items.

- Reducing sizes must be captured largest to smallest. I.e. 30 x 25mm

- Don’t fabricate / guess, only add what can be substantiated via source documents and / or internet and catalogues.

- Kits and Sets are always capture with the work Kit or Set First, then followed by the Type. E.g. Kit Valve Repair, Set Socket Wrench. These will be defined by the Taxonomy.

- Positions are always captured first and then followed by the side E.g. Left Rear = Rear Left

- REBUILD Items, These items are to be created using the material type “ZCAP”. The word “Rebuild” will be part of the description.

- Difference between Wire, Cable, Cord (electrical):
Wire - has a single Core, Generally Bulk
Cable - has multiple Cores with a common insulation, Generally Bulk
Cord - has a termination on one or both ends.

- Difference between Screw and Bolt: 
Screw - cuts its own thread.

Bolt – matches a thread into which it is secured.

- Difference between Lamp, Light, Globe
Lamp – Something that emits light in various colours. 
Light – A complete unit that has the lamp as well as the fittings.

Globe – A spherical dome shape that surrounds a lamp or light.

- Difference between Set vs Kit

Set – A collection of the SAME items with different sizes. I.e. O-Rings, Screws, etc.

Kit – A collection of different items supplied together. Kit Valve Repair may have seals, bearing, seat, etc.

# Solution requirements

The following codification structure shall be used as a guideline to generate descriptions:

### Short Format Description (SFD) (40 Characters)

The MDG will compile the Short Format Description. The rules defined in the Taxonomy tables will determine the Material description (Short) compilation.

- Maximum Length to be 40 characters

- The Classification (Noun) and Sub-classification (Modifiers) are separated by COMMA

- The Sub-classification (Modifiers) and Attribute Values are separated by SEMI-COLON

- The attribute values are separated by COMMA

- If the Sub-classification is “NO MODIFIER”, this will not appear in the final description

- The sequence of the attributes is in the same priority order as defined by the Taxonomy

- If an attribute does not have any value, then the next filled attribute value will be taken. However, if the next attribute value is not be able to fit into the 40-character limit (exceeds 40) then the next filled attribute value will been added in the Short description and henceforth

- No space will be given before or after delimiter

- Modifier will be abbreviated using standard abbreviation list, if the Class is more than 30 characters then Noun also will be abbreviated (Exception)

- Attribute values will be abbreviated using standard abbreviation list.

- The limiting of 40 char is to the nearest attribute value

The MDG will apply the following rules:-

Option 1: NON-OEM ITEMS = Class + Properties.

<NOUN>:<MODIFIER>;<ATTRIBUTE VALUES1>,< ATTRIBUTE VALUES2>,..

E.g. BEARING:ROLR;ID 15 MM,OD 35 MM,WD 11 MM

Option 2: OEM ITEMS = Class + Manufacturer Short Name (see list below) + Manufacturer Part no.

<PRODUCT NAME>,<Ref Value = OEM details, Dwg details>,..< ATTRIBUTE VALUES2>

E.g. WASHER,SPR,CUM,I7564935

### Original Equipment Manufacturer (OEM) List

OEM's are those defined in SAP where the Search Term2 = OEM and the vendor code is "3XXXX"

This list of OEM's is dynamic and changes as defined and approved by the Global Taxonomy Steering Committee.

Any recommended changes / additions / deletions to these approved OEM's needs to be submitted to the GBPO Taxonomy in a format request (MDO). These will then be presented at the weekly Taxonomy Steercom for approval.

### LONG DESCRIPTION FORMAT

The MDG will compile the Long Description. The rules defined in the Taxonomy tables will determine the PO Text compilation.

ALL Long Description:-

- There will be NO abbreviations used in the long description. (Exception for material names only. I.e. Ethylene Propylene Diene Monomer = EPDM)

- The characteristic name will precede the value. I.e. Inside Diameter 25mm

- The drop down list must be used to choose the desired values. If the value desired is not available – a request will need to be made to have it added.

- The part number and its manufacturer (not abbreviated will follow the description. I.e. Cummins not CUM) The MDG will compile the description automatically and use the relevant information accordingly.

- Any Glencore drawings will be part of the long description.

- Long description is a concatenation of the Classification (Noun), Sub-classification (Modifiers) and of all the attributes

- Long description is generated using the following guidelines

  - The Classification (Noun) and Sub-classification (Modifiers) are separated by COMMA

  - The “NO MODIFIER”, this will not appear in the final description

  - The Sub-classification (Modifiers) and Attribute Name are separated by SEMI COLON

  - The Attribute name and Attribute value are separated by COLON.

  - The Attribute value and corresponding Attribute name are separated by COMMA

  - Class (Noun:Modifier) will be abbreviated using standard abbreviation list, if the Class is more than 30 characters or wherever applicable

  - Attribute values will be abbreviated using standard abbreviation list which is more than 30 characters or wherever applicable

  - If attribute value is missing, then long description with attribute name label will not be added and next available attribute value will be followed

  - No space will be given before or after delimiter

NON-OEM ITEMS

<NOUN>,<MODIFIER>;<ATTRIBUTE NAME1:ATTRIBUTE VALUES1>, <ATTRIBUTE NAME2:ATTRIBUTE VALUES2>,..<ATTRIBUTE NAME N:ATTRIBUTE VALUES N>, <REFERENCE TYPE 1:REFERENCE VALUE 1>,….<REFERENCE TYPE N:REFERENCE VALUE N>

E.g. BEARING,ROLLER;INSIDE DIAMETER:15 MM,OUTSIDE DIAMETER:35 MM,WIDTH:11 MM,CAGE MATERIAL:BRASS,MNFR:SKF, PTNO:6206-2RS

OEM ITEMS

<PRODUCT NAME:PRODUCT VALUE>,<ADDITIONAL FEATURES:ATTRIBUTE VALUE>,<OEM MFR NAME>,<OEM P/N>,<REFERENCE TYPE 1:REFERENCE VALUE 1>,….<REFERENCE TYPE N:REFERENCE VALUE N>

E.g. WASHER, MNFR: CUMMINS, PTNO: I7564935

### Class

The class is a combination of the Noun and Modifier within the Taxonomy of the MDG.

To accommodate the 40 character limitation within SAP for the Short description, the Noun will not be abbreviated however the Modifier may be. The modifier abbreviation will be defined within the taxonomy to ensure consistency.

### Reference Text

Every material will be assigned 2 classes

- Technical class: refer taxonomy

- Reference Text: with the following characteristics

| Characteristic Name | Characteristic Value (example) | Data list |
| --- | --- | --- |
| Manufacturer Name | SKF | Multiple Value |
| Manufacturer Part Number | 6206-2RS | Multiple Value |
| Drawing Number | K-12362-CSD | Multiple Value |
| Drawing Version | V1.3 | Multiple Value |
| Item Number | 422 | Multiple Value |
| Vendor Name | ABB | Multiple Value |
| Vendor Part Number | ABB-1236 | Multiple Value |
| Equipment Type | CENTRIFUGAL PUMP | Single value |
| Equipment Manufacturer | KSB | Single value |
| Equipment Model | KSB-1136H | Single value |
| Equipment Serial | 1123AA213 | Single value |
| Not Part of the Description |  |  |

### PART NO

- The Newest OEM Part Numbers will be in the Short Description with the OEM abbreviation directly after the Class. The OEM abbreviation used is defined in the OEM list above (Original Equipment Manufacturer (OEM) List). I.e SIE E5S1234567. (This action is defined in the MDG parameters.)

- Non-OEM materials are divided into those associated to approved Manufacturers (Search Term2 = "MNFR" and the vendor code in SAP = "3XXXX" (Excluding OEM's) and Supplier/Vendors (Search Term2 is not either "MNFR" or "OEM" and the vendor code - "1XXXX"

- Non-OEM’s (MNFR) part numbers and the manufacture name will be appended to both the short description and PO Text after the Class. - This action is defined in the MDG parameters.

- Non-OEM’s (Supplier/Vendor) part numbers and the manufacture name will be appended to both the short description and PO Text after the technical descriptions. - This action is defined in the MDG parameters.

- Internal Manufactured parts that are manufactured iaw a Glencore drawing will have the drawing number captured in the Drawing number fields and appended to the Long description.

### DRAWING NO

- These are always added to the Long description at the end of the Description.

- Internal Glencore Drawings numbers are the only drawings to be used in the short description.

  - If items don’t have a part number and manufacturer but only an internal drawing number, these may be added in the short description following the Class.

- Remember drawings are for a specific item to be made, regardless of the Material.  The material must be captured as part of the description (Value) in both the Short and Long Description before the drawing no.

### CHARACTERISTICS

Characteristics is information that defines the item more specifically. Characteristics are part of the taxonomy and are defined within classification accordingly.

### ADDITIONAL FEATURES

Refers to something important that we want to add and which is not covered in the Characteristics (template).

Example, we have a motor which needs to be supplied with a Spur gear.  This is normally captured in the long descriptions with the Prefix C/W (comes with).

This information that is required in the descriptions but for which there is no characteristics into which the information can be captured, is to be captured in the characteristic “Additional Features”.

There are 3 “Additional Features” characteristics per template. One each for English (EN), Spanish (ES) and French (FR).

### CLEAN-UP OF ADDITIONAL FEATURES DATA

This is any information left in the original description after the reference data and characteristic values have been extracted to the Characteristic value fields. This is populated in the ADDITIONAL FEATURES characteristic.

After the value extraction is completed the ADDITIONAL FEATURES is inspected for the following:

- On items where original data is supplied for a Class Name and the characteristic for such data does not exist in the template the data will remain in the ADDITIONAL FEATURES.

- All data still left over in the ADDITIONAL FEATURES, will be cleaned according to the following guidelines:

- Remove any foreign/special characters from the ADDITIONAL FEATURES, like for example two commas next to each other, string of “*”’s, etc.

- Remove any information that has already been populated in the reference data extraction process and from allocating a Class Name. This will only apply to the material item itself and not to any subset, higher assembly, parts, etc.

- Remove the Characteristic values from the ADDITIONAL FEATURES that have been extracted and populated to the Characteristic value fields in the Template.

- Unnecessary spaces, commas, etc will be removed.

### SAP MATERIAL GROUP and VALUATION CLASS

Note there is a relationship between the Material group and the Valuation class.

The Valuation class is the first two numbers of the material group followed by “00”.

Note that all material groups 0000 to 5999 are for materials. 6000 and above are reserved for Services.

Below is the comprehensive list of all the material groups and the associated valuation class. (This list is dynamic and may change due to requirements.)

| Mat Group | Val Class | Description |
| --- | --- | --- |
| 1000 | 1000 | Capital Acquisitions (Material) |
| 1500 | 1500 | Building Materials |
| 1510 | 1500 | Metal plates including Steel, Iron, etc |
| 1520 | 1500 | Cement / Fly Ash |
| 1530 | 1500 | Cement mixers |
| 1540 | 1500 | Nuts, Bolts, Pins, Washers and other Fasteners. |
| 1550 | 1500 | Bushings including Brackets, Rubber Dampers and Bushes. Excluding Bearings – see 5620 |
| 1600 | 1600 | Electrical Parts including Switches |
| 1610 | 1600 | Generators and Accessories Including Alternators for vehicles |
| 1620 | 1600 | SX-EW Spares |
| 1630 | 1600 | Electrical Motors |
| 1640 | 1600 | Electrical Cables including Ties, Ferrules, Cable connectors, Electric wiring general use etc. |
| 1650 | 1600 | Contactors and Breakers including Fuses and Circuit Breakers |
| 1700 | 1700 | Explosives & Accessories |
| 1800 | 1800 | Cobalt Drier & Accessories |
| 1805 | 1800 | DMS plant spares |
| 1810 | 1800 | Stainless Steel Blanks |
| 1815 | 1800 | Rolled Lead Anodes |
| 1820 | 1800 | Acid plant spares |
| 1825 | 1800 | All gaskets |
| 1830 | 1800 | Tanks and Accessories |
| 1835 | 1800 | Agitators and Accessories. |
| 1840 | 1800 | Thickener Spares |
| 1845 | 1800 | Cyclone spares |
| 1850 | 1800 | Vibratory spares and Accessories |
| 1900 | 1900 | Fuel, Gas |
| 1910 | 1900 | Propane |
| 2000 | 2000 | Ground Support Supplies |
| 2100 | 2100 | Fuel - Power Generation |
| 2200 | 2200 | Pipes - Metallic & Fittings including related items but excluding hydraulic pipes and fittings – see 5910 |
| 2210 | 2200 | Pipes - Non-Metallic & Fittings including Hoses, Couplings not marked as hydraulic or metal. – see 5910 |
| 2220 | 2200 | Pipes - Victaulic & Fittings |
| 2230 | 2200 | All Seals and associated items |
| 2240 | 2200 | Valves |
| 2500 | 2500 | Abrasives |
| 2600 | 2600 | Drills, Bits & Steel |
| 2610 | 2600 | Vee, Fan and Drive Belts Including brackets directly associated |
| 2620 | 2600 | Welding Supplies |
| 2700 | 2700 | Office Furniture & Supplies |
| 2800 | 2800 | Oils, Greases, Lubricants |
| 2900 | 2900 | Fire Protection |
| 2920 | 2900 | PPE & Safety Supplies |
| 3000 | 3000 | Pump Parts & Accessories |
| 3010 | 3000 | Water Supply System |
| 3020 | 3000 | Compressors and Accessories |
| 3300 | 3300 | Ground Engagement Tools - Tips, Blades, Buckets, Edges etc. |
| 3600 | 3600 | Tyres / Tires |
| 3700 | 3700 | Surface Equipment Parts |
| 3710 | 3700 | Tracks & Accessories - Links, Rollers, Track Pads and Parts |
| 3720 | 3700 | ADT machine spares and accessories |
| 3730 | 3700 | CATERPILLAR accessories and spare parts - For body parts and non-specific parts (not listed separately) |
| 3740 | 3700 | Hitachi accessories and spare parts - For body parts and non-specific parts (not listed separately) |
| 3750 | 3700 | Drill Rig Spares & Accessories - For body parts and non-specific parts (not listed separately) |
| 3760 | 3700 | Volvo accessories and spare parts - For body parts and non-specific parts (not listed separately) |
| 3765 | 3700 | P&H accessories and spare parts |
| 3770 | 3700 | Komatsu accessories and spare parts |
| 4000 | 4000 | Timber, Lumber, Nails |
| 4005 | 4000 | Small Tools & Supplies - mostly consumables or minor tools e.g. Chisels, drill bits etc. |
| 4010 | 4000 | Hand tools - The tool itself, incl specialized manufacturer tools e.g. CAT |
| 4015 | 4000 | Aircon and accessories - For vehicle aircon specific items |
| 4020 | 4000 | Cleaning materials |
| 4025 | 4000 | Fencing |
| 4030 | 4000 | Foodstuffs |
| 4035 | 4000 | Garden Equipment & Accessories |
| 4040 | 4000 | Hospital & First Aid Supplies |
| 4045 | 4000 | Packaging and storage accessories |
| 4050 | 4000 | Stationery |
| 4055 | 4000 | Big Bags & Other packaging |
| 4060 | 4000 | Pest control |
| 4065 | 4000 | Paint |
| 4070 | 4000 | Boiler Spares |
| 4075 | 4000 | Plumbing |
| 4080 | 4000 | Scaffolding |
| 4090 | 4000 | Other misc Supplies |
| 4100 | 4100 | Lighting and Accessories - Lamps and general Light Fittings (Vehicle specific items to individual vehicle types and EMV) |
| 4200 | 4200 | Laboratory Supplies |
| 4300 | 4300 | Ore Refinement Equipment Supplies |
| 4400 | 4400 | Other Refining Equipment Supplies |
| 4500 | 4500 | Computer Software & Hardware |
| 4600 | 4600 | Water Treatment Equipment/Chemicals |
| 4700 | 4700 | Refractory Products |
| 4800 | 4800 | Locomotive, Rail and Track materials |
| 4900 | 4900 | Wire Ropes and Chains including Crane Cables |
| 5000 | 5000 | U/G Equipment Parts |
| 5010 | 5000 | Ventilation & Accessories |
| 5100 | 5100 | Instrumentation - Controls, Solenoid, PLC's ECU's all items related to equipment control, meters mechanical and digital and plant specific equipment i.e. Siemens |
| 5110 | 5100 | Digital Scales and Accessories |
| 5300 | 5300 | Bulk Oxygen |
| 5305 | 5300 | Carbon |
| 5310 | 5300 | Cyanide |
| 5315 | 5300 | Lime |
| 5320 | 5300 | Caustic Soda |
| 5325 | 5300 | Flocculent |
| 5330 | 5300 | Nitric Acid |
| 5335 | 5300 | CalFoam Graphite |
| 5340 | 5300 | Diluent - Shellsol |
| 5345 | 5300 | Extractent - LIX |
| 5350 | 5300 | FeSi (Ferrosilicon) |
| 5355 | 5300 | SMBS (Sodium Metabisulfite) |
| 5360 | 5300 | SO2 / Sulphur Dioxide |
| 5365 | 5300 | Sulphuric Acid |
| 5370 | 5300 | Magnesium Oxide |
| 5375 | 5300 | Sulphur |
| 5380 | 5300 | Other chemicals and reagents |
| 5385 | 5300 | Magnesium Hydroxide |
| 5390 | 5300 | Sodium Hydroxide |
| 5395 | 5300 | Hydrochloric Acid |
| 5400 | 5400 | Muriatic Acid |
| 5405 | 5400 | Hydrogen Peroxide |
| 5500 | 5500 | Grinding media #2 Mill-Slugs and Balls |
| 5610 | 5600 | Transformers |
| 5620 | 5600 | Power transmission and Bearings - General use Gearboxes, pulleys, tire and shaft connectors, drive shafts, sprockets, clutches, universal joints, shafts on drive train, differentials, bearing related items like cups, cones, brackets etc. (All Vehicle specific to individual vehicles types) |
| 5700 | 5700 | Conveyor spare parts and accessories |
| 5750 | 5700 | Assays |
| 5800 | 5800 | Mill Liners & Supplies |
| 5810 | 5800 | Crushing equipment and accessories |
| 5820 | 5800 | Mill spare parts and accessories |
| 5900 | 5900 | Brakes - All brake related items including pads, brake cylinders, valves, cylinders etc. |
| 5905 | 5900 | Filters - including elements and fittings, driers, fuel water separators |
| 5910 | 5900 | Hydraulic Hoses & Accessories - Only hydraulic hoses and accessories, other hydraulic items like pumps fall under the equipment. Excluding Filters and seals. |
| 5915 | 5900 | Light Vehicle Parts - For body parts and non-specific parts (not listed separately), vehicle specific parts |
| 5920 | 5900 | Mechanical Parts - Only use in exceptional cases where mechanical parts can't be allocated to e.g. CAT |
| 5925 | 5900 | Road Maintenance |
| 5930 | 5900 | Camp equipment |
| 5935 | 5900 | Crane Spares and accessories - For body parts and non-specific parts (not listed separately), all mobile cranes Terrex, Tadano, Zoom Lion |
| 5940 | 5900 | Geological and survey equipment |
| 5945 | 5900 | Skid Steer Loader accessories and spares - For body parts and non-specific parts (not listed separately), Bobcats |
| 5955 | 5900 | Other mobile equipment spare parts - For body parts and non-specific parts (not listed separately) for trucks, JCB and CAT etc. forklifts |
| 5960 | 5900 | Security Equipment |
| 5965 | 5900 | Hoist Parts - Hoist blocks, cables, attachments to hoisting gear |

### MATERIAL TYPE

| Material Type | Description |
| --- | --- |
| ERSA | Spare Parts – Spare parts are used to replace defective parts. They may be kept in stock with stock levels. A Material Master record of this material type will contain basic data, purchasing data, Plant/Storage data, MRP and accounting data. It can be an integral part for maintenance bills of materials (BOMs) an example of a spare part would be a specific hydraulic oil filter for a truck or loader. This will also encompass parts used in manufacturing that are replace or repairable. Spare parts can be serialised with system generated internal Number rang. Consumables and other items (e.g. Welding Rods, PPE, Clothing, etc.) will be created as a material ERSA |
| HIBE | Operating Supplies – Operating supplies are all reagents used in the process Plant. Fuels & Gases used in operations. Mill Ball and Grinding Media used in operations. For Example Reagents used to create other products i.e. Sulphur used to create Sulphuric Acid. |
| UNBW | Non-valuated materials – Low value Capital Items – Non valuated materials managed on a quantity basis, but not by value. Insurance spares will be created with Material type UNBW |
| ZCAP | Capital Spares – Capital spares will be similar to spare parts in characteristics, however they will be created with separate material type in order to differentiate materials from regular spare parts. Capital spare Material will be valuated and stock items, like spare parts. A material master record of this material type will contain basic data, purchasing data, plant/storage data, MRP and accounting data. As a business rule, all Refurbishable items will be created as capital spares with Split Valuation. Capital Spares can be serialised with eternal number range. Since separate material type will be provided for Capital spares, setting “Critical Part” indicator becomes optional. |
| IBAU | Maintenance Assembly Materials |
| VERP | Packaging Materials |
| ZPRJ | Project Materials |
| ZZMT | MAIT Integration Materials |

### LANGUAGE STANDARD

Glencore as an international company uses English as the official language for all communications. This has been implemented within all SAP and MDG environments.

There are many words that in English are spelt differently depending on whether USA or UK English is being used. Below are a few examples.

| English (USA) | English (UK) |
| --- | --- |
| Aluminium | Aluminum |
| Molded | Moulded |
| Adaptor | Adapter |
| Color | Colour |
| Tire | Tyre |
| Arbor | Arbour |
| Labor | Labour |
| Standardize | Standardise |

### SPECIAL CHARACTERS

Within the descriptions being compiled via the taxonomy in the MDG, there are specific rules defined to ensure data consistency. Although the MDG will apply these automatically when compiling the various descriptions, these rules need to be applied when entering the values in the MDG.

| SPECIAL CHARACTERS | RULES FOR SPECIAL CHARACTERS |
| --- | --- |
| . | Full stops will be used to break decimals. No spaces are allowed on either side of a full stop. No full stop are to be at the end of description. |
| / | A Forward (front) slash will only be used in Part numbers; Drawing numbers; Reference numbers or Fractions. No spaces are allowed on either side of a Front Slash. It is also used to separate values within a property (Used instead of the word “or”). I.e. 110 or 220 VAC = 110/220 VAC |
| - | A hyphen (dash) is allowed in the use of Part Numbers, Drawing numbers; Reference numbers or where grammatically necessary in descriptions. It is also used to indicate a Negative (minus) value. It is further used to indicate a range of values. (Used instead of the word “to”). I.e 110 to 220VAC = 110-220VAC No spacing allowed on either side. |
| x | An alone standing X is only allowed to denote "times" in sizes and will be in UPPER case. I.e. 20 X 50 X 10mm. If an X is part of the keyword or brand it MUST be UPPERCASE. |
| & | Ampersand may be used to replace the word AND to save space |
| % | Percentage may NOT be used to describe proportion. The abbreviation "PCT" is to be used instead. |
| + | A Plus is allowed and us typically used in Tolerance. |
| +/- | A Plus Minus is allowed and is typically used in Tolerance. Do not use “±”. Note that ASCII code characters are not to be used. |
| ≥ | The "Greater than or Equal to" symbol is to be captured as ">=" Note that ASCII code characters are not to be used. |
| ≤ | The "Less than or Equal to" symbol is to be captured as "<=" Note that ASCII code characters are not to be used. |
| ° | The Degree sign is to be captured as DEG Note that ASCII code characters are not to be used. |
| “ | This is commonly used in industry to denote INCHES. This is not to be used. The INCH abbreviation “IN” is to be used instead. |
| ‘ | This is commonly used in industry to denote FEET/FOOT. This is not to be used. The FEET/FOOT abbreviation “FT” is to be used instead. |
| * | This symbol is often used to separate different dimensions / values in industry. This symbol is NOT to be used. The “X” is to be used. Eg. 150*100 is to be captured as 150 X 100. |
| Ø | This is very often used to denote “Diameter” or “Bore” and is not to be used. Use the prefix “DIA” instead. Note that ASCII code characters are not to be used. |
| MM² / MM³ | The miniature “²” or “³” are not to be used. MM2 or MM3 is the correct manner of capturing these values. Note that ASCII code characters are not to be used. |
| (3) DIA 10 MM HOLES | When there is more than a single quantity of a given value, the quantity is to be in brackets followed by the value. I.e. (3) DIA 10 MM HOLES means that there are 3 holes and the holes are 10 MM in diameter. |

### TOLERANCES

When Capturing Tolerances the following format is to be used.

Plus: the “+” shall be used, e.g. +0.001

Minus: the “-” shall be used, e.g. -0.0001

Plus/minus: “+/-“ shall be used, e.g. +/-0.0001

### SERIES OF NO’S

Apply the following rules for where values are capture in series. This is normally used for Bits, Sockets, Kits, etc.

| Single | 1,3,9,15,18 |
| --- | --- |
| Range | 1-3 -10 TO 15 |
| Single and Range combination | 1,3,7-9,15,18,20-25 |

### RANGES

Please apply the following rules when capturing ranges in value fields.

Note that when a range is from a negative to a positive then the word ”to” is used.

| Negative to Positive | -150 TO 120 |
| --- | --- |
| Positive to Positive | 120-150 |
| Minimum to Maximum | 120-150 |

### COLOR

When capturing colour(s) of items it is normally done alphabetically. E.g. Blue, Green Dark, Green Light, Yellow.

Colors are always captured with the Color name first followed by the shade. E.g. Green Dark

NOTE: This rule is NOT APPLIED to items where the color code defines ratings. E.g. Resistors, Pipes, Gaskets etc.  These colors sequences must not be changed and must be captured as per the Manufacturer specifications.

### DIMENSIONS, SIZES

Prefixes are added to Descriptions. E.g. ID 25 X OD 35 MM X LG 6 M

Standard Abbreviations

| ABBREVIATION - EN | ABBREVIATION - ES | ABBREVIATION - FR | DESCRIPTION |
| --- | --- | --- | --- |
| DIA | DIA | DIA | DIAMETER |
| DP | PROF | PROF | DEPTH |
| HT | LAT | HT | HEIGHT |
| ID | DIA INT | DIA | INSIDE DIAMETER |
| OD | DIE EXT | DE | OUTSIDE DIAMETER |
| LG | LRG | LG | LENGTH |
| THK | ESP | EP | THICKNESS |
| SQ | CUAD | CAR | SQUARE |
| WD | LRG | LRG | WIDTH |

Format examples: Note the sequence in which these dimensions have been captured.

| DIMENSION / SIZE FORMAT | EXAMPLE |
| --- | --- |
| DIA X HT X DP | DIA 25 X HT 15 X DP 50 MM |
| DIA X LG | DIA 1/2 X LG 10 IN |
| DIA X LG X THK | DIA 50 X LG 200 X THK10 MM |
| DIA X THK | DIA 10 X THK 1/2 IN |
| ID X OD | ID 3 X OD 4-3/4 IN |
| ID X OD X LG | ID 12-1/2 X OD 15 X LG 24 IN |
| ID X OD X LG X THK | ID 10 X OD 20 X LG 150 X THK 1.5 MM |
| ID X OD X THK | ID 3/4 X OD 1-1/8 X THK 1/4 IN |
| SQ X DP | SQ 50 X DP 200 MM |
| SQ X DP X THK | SQ 50 X DP 25 X THK 2 MM |
| SQ X LG | SQ 12 X LG 20 IN |
| SQ X HT | SQ 3/4 X HT 4-5/16 IN |
| SQ X THK | SQ 65 X THK 45 MM |
| WD X LG | WD 1-1/2 X LG 2-3/4 M |
| WD X LG X DP | WD 250 X LG 800 X DP 50 MM |
| WD X LG X HT | WD 3-1/2 X LG 12 X HT 2 IN |
| WD X LG X THK | WD 35 X LG 1200 X THK 4 MM |

### UNIT OF MEASURES/NOTES

(These are NOT BUOM, but rather value UOM's)

Container Type

| ABBREVIATION | DESCRIPTION |  |
| --- | --- | --- |
| BG |  | Bag |
| BT |  | Bottle |
| BX |  | Box |
| CS |  | Case |
| CT |  | Carton |
| DR |  | Drum |
| DZ |  | Dozen |
| EA |  | Each |
| GR |  | Gross |
| JR |  | Jar |
| PD |  | Pad |
| PK |  | Pack |
| PL |  | Pallet |
| PR |  | Pair |
| TB |  | Tube |

Digital

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| GB | Gigabyte |
| KB | Kilobyte |
| MB | Megabyte |
| TB | Terabyte |

- Electrical Units/Ratings

When capturing electrical values the general order is as follows.  (This is used when there is no Value field to be able to capture values per characteristic).

If you have 2 different values for the same type of Voltage/Amp etc. always capture the Lowest First

Eg.: 120/240VAC

Volts, Amps, Phase, Watts, VA Rating, followed by the rest (Horsepower, Ohm, etc.), Hertz is always last.

I.e: 120/240VAC, 5A, 2PH, 50W, 3KV, 3HP, 5 OHM, 50/60HZ

There are, however items where the Electrical rating, should be split into separate Properties, as below:

- CURRENT = Ampere (A)

- POTENTIAL = Voltage (V)

- POWER = Wattage (W)

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| A | Ampere |
| AH | Ampere hour |
| AS | Ampere second |
| AC/DC | Alternating current/direct current |
| C | Coulomb |
| F | Farad |
| H | Henry |
| KA | Kilo Ampere |
| KV | Kilovolt |
| KVA | Kilo volt ampere |
| KW | Kilowatt |
| KWH | Kilowatt-hour |
| OHM | Ohm |
| VA | Volt-ampere |
| W | Watt |
| KW | Kilo Watt |

- Voltage units

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| V | Volt |
| VAC | Volt Alternative Current |
| VDC | Volt Direct Current |

- Frequency

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| GHZ | Gigahertz |
| HZ | Hertz |
| KHZ | Kilohertz |
| MHZ | Megahertz |

- Length

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CM | Centimetre (Only to be used for Clothing related dimensions) |
| FT | Feet |
| IN | Inch |
| KM | Kilometre |
| LG | Length |
| M | Meter |
| MI | Mile |
| MM | Millimetre |
| RL | Roll |
| YD | Yard |
| UM | Micron |

Note:

Less than 1000 MM = MM

Greater than 1000 MM = M

Greater than 1000 M = KM

- Nominal Pipe Sizes

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| SCH | Schedule |
| NPS | Nominal Pipe Size |

- Power Unit

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| HP | Horsepower |
| W | Watt |

- Pressure Ratings

DO NOT Convert PSIG, PSID or PSIA Ratings. 
DO NOT Convert PSI to LB for Pressure Gauges, Switches and Transmitters.
DO NOT Convert PSI to PSIG

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| BAR | Bar |
| HPA | Hecto Pascal |
| KPA | Kilo Pascal |
| MPA | Mega Pascal |
| MBAR | Millibar |
| PA | Pascal |
| PSI | Pounds per Square Inch |
| PSID | Pounds per Square Inch Differential |
| PSIG | Pounds per Square Gauge |

- Shape

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| RND | Round |
| SQ | Square |
| TRIA | Triangle |
| RECT | Rectangle |
| HEX | Hexagon/Hexagonal |
| OCT | Octagon |
| HEP | Heptagon |

- Speed

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CPM | Counts per minute |
| CW | Continuous wave |
| FT/MIN | Foot per minute |
| FT/SEC | Foot per second |
| KM/H | Kilometre per hour |
| MPH | Mile per hour |
| M/S | Meter per second |
| RPM | Revolutions per minute |
| RPS | Revolutions per second |

- Time

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| D | Day |
| HR | Hour |
| MIN | Minutes |
| MS | Millisecond |
| S | Second |
| YR | Year |
| MNTH | Month |

- Temperature Unit

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| DEG C | Degree Celsius |
| DEG F | Degree Fahrenheit |
| K | Kelvin |

- Volume

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CL | Centilitre |
| DL | Decilitre |
| DM3 | Cubic decimetre |
| DR | Drum |
| FL OZ | Fluid ounce |
| FT3 | Cubic foot |
| GAL | Gallon |
| L | Litre |
| M3 | Cubic meter |
| ML | Millilitre |
| MM3 | Cubic millimetre |

- Weight

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CG | Centigram |
| DG | Decigram |
| G | Gram |
| HG | Hectogram |
| KG | Kilogram |
| LBS | Pound |
| OZ | Ounce |
| T | Metric ton (UK) |
| TON | Short ton (US) |
| TES | Tesla |
| MG | Milligram |
| ST | Stone |

### THREAD ABBREVIATION and NOTES

Note: If there is a sequence to capture for the thread information Capture it in a string format.
Capture as Thread Direction, Series, Class, Thread Size, Thread type.

Imperial Example

A typical example and breakdown of how thread data may be found is: 1/4-20UNC-2A,

- LH = if Left Hand (No symbol required for right hand thread).

- UNC = Thread series (Unified Coarse thread).

- 2A = Thread class symbol (A = External Tread and B = Internal Thread)

- 20 = Number of Threads Per Inch (TPI).

- 1/4 = Nominal Size.

Short Description. ex. LH,UNC,2A,20,1/4 IN
Long Description ex. LH,UNC,Cl 2A,20 TPI,1/4 IN

Metric Example

A typical example and breakdown of how thread data may be found is: M10, 1.25mm BSF, 2g

- LH = if Left Hand (No symbol required for right hand thread).

- BSF = Thread Type (British Standard Fine)

- 1.25mm = Thread Pitch

- 2g = Thread Class (Capital Letter = Internal, Small Letter = External Thread)

- M10 = Nominal size

Short Description. ex. LH,BSF,2G,1.25 MM,M10
Long Description ex. LH,BSF,CL 2G,PITCH 1.25 MM,M10

Thread Class Imperial: 
A = External

B = Internal

Thread Class metric:

G/H = Internal

E/F = External

Thread Direction: 
RH = Right Hand (this is normally not captured as this is normally the default direction)
LH = Left Hand

| THREAD SERIES LIST |  |
| --- | --- |
| ABBRREVISION | DESCRIPTIONS |
| BSF | British Standard Fine |
| BSP | British Standard Pipe |
| BSPF | British Standard Pipe Fitting |
| BSPP | British Standard Pipe Parallel |
| BSPT | British Standard Pipe Taper |
| BSW | British Standard Whitworth |
| FBSP | Female British Standard Pipe |
| FNPT | Female National Pipe Taper |
| FPT | Female Pipe Taper |
| MPT | Male Pipe Taper |
| NPS | National Pipe Straight |
| NPT | National Pipe Taper |
| NST | National Standard Thread |
| UNC | Unified National Coarse |
| UNEF | Unified National Extra Fine |
| UNF | Unified National Fine |
| UNR | Unified National Round (Round Root) |
| UNS | Unified National Special |

### MATERIAL ABBREVIATION

| ABBREVIATION | DESCRIPTION |  |
| --- | --- | --- |
| AG |  | Silver |  |
| AL |  | Aluminum |  |
| AL ALOY |  | Aluminium Alloy |  |
| APL |  | Aluminium Polyethylene Laminate |  |
| AS |  | Alloy steel |  |
| SAN |  | Styrene-Acrylonitrile (plastic) |  |
| ASB |  | Asbestos |  |
| AU |  | Gold |  |
| BMS |  | Bright mild steel |  |
| BRS |  | Brass |  |
| BRS CR PLTD |  | Brass Chrome Plated |  |
| BRS FRGD |  | Brass Forged |  |
| BRS NI PLTD |  | Brass Nickel Plated |  |
| BRS PLTD |  | Brass Plated |  |
| BRS SN PLTD |  | Brass Tin Plated |  |
| BRZ |  | Bronze |  |
| C |  | Carbon |  |
| CA |  | Cellulose Acetate |  |
| CANV |  | Canvas |  |
| CD |  | Cadmium |  |
| CHN STL |  | Chain Steel |  |
| CI |  | Cast iron |  |
| CI HRD NI |  | Cast Iron Hard Nickel |  |
| CEMNT |  | Cement |  |
| COMPD |  | Compound |  |
| CNCRT |  | Concrete |  |
| CPE |  | Chlorinated Polyethylene |  |
| CPVC |  | Chlorinated Polyvinyl chloride |  |
| CR |  | Chrome/chromium |  |
| CRMO |  | Moly chrome/chrome moly |  |
| CR STL |  | Chrome steel |  |
| CS |  | Carbon steel |  |
| CS CAD |  | Carbon Steel Cadmium |  |
| CS CAD PLTD |  | Carbon Steel Cadmium Plated |  |
| CS CTD |  | Carbon Steel Coated |  |
| CS CU PLTD |  | Carbon Steel Copper Plated |  |
| CS GALV |  | Carbon Steel Galvanized |  |
| CS PLTD |  | Carbon Steel Plated |  |
| CS RUBR LND |  | Carbon Steel Rubber Lined |  |
| CS GALV |  | Carbon Steel, Galvanized |  |
| CS PP LND |  | Carbon Steel Polypropylene Lined |  |
| CS ZN CTD |  | Carbon Steel Zinc Coated |  |
| CS ZN PLTD |  | Carbon Steel Zinc Plated |  |
| SS CST |  | Cast Stainless steel |  |
| CU |  | Copper |  |
| CU ALOY |  | Copper Alloy |  |
| CU ALOY CST |  | Copper Alloy Cast |  |
| CU NI ALOY |  | Copper Nickel Alloy |  |
| CU NI PLTD |  | Copper Nickel Plated |  |
| CU SN PLTD |  | Copper Tin Plated |  |
| CVS |  | Chrome Vanadium Steel |  |
| DI |  | Ductile iron |  |
| EPTFE |  | Expanded Polytetrafluoroethylene |  |
| EP |  | Ethylene Propylene |  |
| EPDM |  | Ethylene Propylene Diene Monomer |  |
| EPM |  | Ethylene Propylene Monomer |  |
| EPR |  | Ethylene Propylene Rubber |  |
| ETFE |  | Ethylene-tetrafluoroethylene |  |
| EVA |  | Ethylene Vinyl Acetate |  |
| FBR |  | Fibre |  |
| FBRGLS |  | Fiberglass |  |
| FE |  | Iron |  |
| FE GALV |  | Iron Galvanized |  |
| FE NI PLTD |  | Iron Nickel Plated |  |
| FE OXID |  | Iron Oxide |  |
| FEP |  | Fluorinated Ethylene Propylene |  |
| FLPE |  | Fluorinated Polyethylene |  |
| GALV |  | Galvanized |  |
| GALV CTD |  | Galvanized Coated |  |
| GE |  | Germanium |  |
| GI |  | Galvanised iron |  |
| GLS SFTY |  | Glass Safety |  |
| GMS |  | Galvanised Mild Steel |  |
| GRP |  | Glass Reinforced Plastic |  |
| GS |  | Galvanised steel |  |
| HCS |  | High Carbon Steel |  |
| HDPE |  | High Density Polyethylene |  |
| HDPP |  | High Density Polypropylene |  |
| HDPU |  | High Density Polyurethane |  |
| HG |  | Mercury |  |
| HNBR |  | Hydrogenated Acrolyontrile Butadiene Rubber |  |
| HRS |  | Hot Rolled Steel |  |
| HS |  | Hassium |  |
| HSS |  | High Speed Steel |  |
| HTS |  | High Tensile Steel |  |
| LCP |  | Liquid Crystal Polymer |  |
| LDPE |  | Low Density Polyethylene |  |
| LTHR |  | Leather |  |
| LTS |  | Low Tensile Steel |  |
| MDPE |  | Medium Density Polyethylene |  |
| MG |  | Magnesium |  |
| MGO |  | Magnesium Oxide |  |
| MI |  | Malleable Iron |  |
| MN |  | Manganese |  |
| MO |  | Molybdenum |  |
| MPTFE |  | Multifull Polytetrafluoroethylene |  |
| MS |  | Mild Steel |  |
| MS EPXY CTD |  | Mild Steel Epoxy Coated |  |
| MS GALV |  | Mild Steel Galvanized |  |
| NBR |  | Nitrile Butadiene Rubber |  |
| NI |  | Nickle |  |
| NI HRD |  | Nickel Hard |  |
| NI PLTD |  | Nickel Plated |  |
| NI PLTD CU |  | Nickel Plated Copper |  |
| NI STL |  | Nickel Steel |  |
| NPR |  | Neoprene |  |
| NYL |  | Nylon |  |
| P |  | Phosphorus |  |
| PA |  | Polyamide |  |
| PAN |  | Polyacrylonitrile |  |
| PB |  | Lead |  |
| PC |  | Polycarbonate |  |
| PE |  | Polyethylene |  |
| PET |  | Polyethylene Terephthalate |  |
| PEVA |  | Polyethylene Vinyl Acetate |  |
| PI |  | Polyimide |  |
| PLSTC |  | Plastic |  |
| PLSTC FBR |  | Plastic Fibre |  |
| PLSTC HRD |  | Plastic Hard |  |
| PLTD BRS |  | Plated Brass |  |
| PLSTC FE |  | Plated Iron |  |
| PLSTC STL |  | Plated Steel |  |
| PLYMR |  | Polymers |  |
| PLYF |  | Polyolefin |  |
| PP |  | Polypropylene |  |
| PPO |  | Polypropylene Oxide |  |
| PPS |  | Polypropylene Sulphide |  |
| PT |  | Platinum |  |
| PTFE |  | Polytetrafluoroethylene |  |
| PU |  | Polyurethane |  |
| PVA |  | Polyvinyl Acetate |  |
| PVAL |  | Polyvinyl Alcohol |  |
| PVB |  | Polyvinyl Butral |  |
| PVC |  | Polyvinyl Chloride |  |
| PVCA |  | Polyvinyl chloride acetate |  |
| PVDC |  | Polyvinyl Dene chloride |  |
| PVDF |  | Polyvinyl Dene Fluoride/Difluoride |  |
| PVF |  | Polyvinyl Fluoride |  |
| PVFM |  | Polyvinyl Formal |  |
| PVS |  | Polyvinyl Siloxane |  |
| RH |  | Rhodium |  |
| RPTFE |  | Reinforce Polytetrafluoroethylene |  |
| SBR |  | Styrene Butadiene Rubber |  |
| SGI |  | Spheroidal Graphite Iron |  |
| SHT MTL |  | Sheet Metal |  |
| SHT STL |  | Sheet Steel |  |
| SI |  | Silicone/Silicon |  |
| SIC |  | Silicon Carbide |  |
| SN |  | Tin |  |
| SS |  | Stainless Steel |  |
| STL |  | Steel |  |
| TI |  | Titanium |  |
| TPE |  | Tetrafluroethylene |  |
| TPE |  | Thermoplastic Elastomer |  |
| UPES |  | Unsaturated Polyester |  |
| UPVC |  | Un-practiced polyvinyl chloride |  |
| V |  | Vanadium |  |
| WC |  | Tungsten Carbide |  |
| WI |  | Wrought Iron |  |
| WL |  | Wool |  |
| WS |  | Wrought Steel |  |
| ZN |  | Zinc |  |
| ZR |  | Zirconium |  |

### GENERAL ABBREVIATION

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| AWS | American Welding Association |
| AWG | American Wire Gauge |
| BC | Bayonet Cap (lamp) |
| BTU | British thermal unit |
| CD | Cadela (luminous Intensity) |
| CAL | Calorie |
| CARAT | Carat |
| CA | Centiare |
| Cl | Class |
| CES | Candelabra Edison Screw (lamp) |
| C/W | Come With |
| C | Core (cable) |
| CSK | Countersunk |
| DB | Decibel |
| DEG | Degree / ° |
| DN | Dimension Nominal |
| DP | Double Pole |
| DPDT | Double Pole , Double Throw |
| DPST | Double Pole , Single Throw |
| DWG | Drawing |
| DUPL | Duplicate |
| ELECMAG | Electromagnetic |
| ES | Edison Screw (lamp) |
| F | Farad |
| FF | Flat Face |
| FL OZ | Fluid ounce |
| FT LBS | Foot Pound |
| FR | Front |
| HD | Head |
| HRS | Hours |
| HYDR | Hydraulic |
| G | Gauss |
| GR | Grade (material) |
| GES | Giant Edison Screw (lamp) |
| HEX | Hexagon/Hexagonal |
| HPS | High Pressure Sodium |
| IMP | Impulse |
| J | Joule |
| KJ | Kilojoule |
| LH | Left Hand |
| LBS | Pound |
| LM | Lumen |
| LES | Lilliput Edison Screw (lamp) |
| LDR | Light Dependent (Resistor) |
| LUX | Lux |
| M | Male |
| MNFR | Manufacturer |
| MAX | Maximum |
| MFD | Microfarad |
| MBC | Miniature Bayonet Cap |
| MES | Miniature Edison Screw (lamp) |
| MW HR | Megawatt hour |
| UM | Micron / μm |
| MIN | Minimum |
| MISC | Miscellaneous |
| N | Newton (force) |
| NB | Nominal Bore |
| N/M2 | Newton per meter squared |
| N/S | Newtons per second (rate of force development) |
| NC | Normally Closed |
| NO | Normally Open |
| OEM | Original Equipment Manufacturer |
| PPM | Part per million |
| PCT | Percent |
| PERM | Permanent |
| PCS | Piece |
| PLTD | Plated |
| P | Pole |
| PH | Phase |
| PM | Post Meridiem |
| PDR | Potential Duplicate Report |
| PSIA | Pounds per square inch absolute |
| PRES | Pressure |
| RAD | Radian |
| REAR | Rear |
| RF | Raised Face |
| RH | Right Hand |
| SBC | Small Bayonet Cap (lamp) |
| SCKT | Socket |
| SES | Small Edison Screw (lamp) |
| SFTWR | Software |
| SSR | Solid State (Relay) |
| SUPL | Supplier |
| TEMP | Temperature |
| THD | Thread/Threaded |
| VSD | Variable Speed Drive |
| WH | Watt-hour |
| YEL | Yellow |

### Examples per material (item) types

This section is to supply information for a particular material master type. Please note that not all material types are in examples below.

Material is to be captured as Material and Material Grade E.g. Stainless Steel 316L, further the Material Specification must be captured where available E.g ASTM A193 B8 CL1

### Metric to Imperial Conversion

All imperial sizes are to be fractionalized.

E.g. 1.5 IN = 1-1/2 IN

Here is a good website to use to do conversions –

https://www.metric-conversions.org/length/inches-to-millimeters.htm

Select “Inches to “Millimetre’s”

Imperial size converted to metric in millimetres.

Enter imperial decimal size.

Highlight and copy.

Paste copied millimetre’s value

Click on “Swap Units”

Remove “mm” at the end of number

Imperial fraction is given and to be used.

# Sign-off

I/We certify that this document completely and accurately reflects Glencore Copper Global requirements for this Business Design Document.  I/We understand that anything not explicitly mentioned in this document is considered out of scope for the project.

| Project Role | Signature | Date |
| --- | --- | --- |