Standardization of

Material Descriptions

Production Guidelines

# Basic Rules

The follow rules are to be applied

- Always ask yourself “WHAT DO I HAVE IN MY HAND” – what is the Noun?

- Respect the format proposed for every value you want to add.

- Respect the value standardization.

- Space Allowed. Any multi-spacing within the value range is replaced with a single space.

- Correct all spelling mistakes.

- No punctuation and accent.

- Only some special characters are allowed. See p.10 of this document.

- No plural in the Noun.

- No abbreviation in the Noun.

- No Size Conversions are to be done EG 1-1/2in to 38.1mm or visa versa.

- All imperial sizes to be fractionalised – not decimals. I.e. 1-1/2 not 1.5 inches. Note the format that fractions are capture. (https://www.metric-conversions.org is a great site to use.)

- Use existing abbreviations/acronyms only. See pg.12-23of this document.

- No base unit of measure (BUOM) in the short description.

- All data is to be in upper case.

- Additional Features – Must have direct value add to class.

- Please see below Rules for short description regarding OEM and Non-OEM items.

- Reducing sizes must be captured largest to smallest. I.e. 30 x 25mm

- Don’t fabricate / guess, only add what can be substantiated via source documents and / or internet and catalogues.

- Kits and Sets are always capture with the work Kit or Set First, then followed by the Type. E.g. Kit Valve Repair, Set Socket Wrench. These will be defined by the Taxonomy.

- Positions are always captured first and then followed by the side E.g. Left Rear = Rear Left

- REBUILD Items, These items are to be created using the material type “ZCAP”. The word “Rebuild” will be part of the description.

- Difference between Wire, Cable, Cord (electrical):
Wire - has a single Core, Generally Bulk
Cable - has multiple Cores with a common insulation, Generally Bulk
Cord - has a termination on one or both ends.

- Difference between Screw and Bolt: 
Screw - cuts its own thread.

Bolt – matches a thread into which it is secured.

- Difference between Lamp, Light, Globe
Lamp –Something that emits light in various colours. 
Light –A complete unit that has the lamp as well as the fittings.

Globe –A spherical dome shape that surrounds a lamp or light.

- Difference between Set vs Kit

Set – A collection of the SAME items with different sizes. I.e. O-Rings, Screws, etc.

Kit–A collection of different items supplied together. Kit Valve Repair may have seals, bearing, seat, etc.

### SPECIAL CHARACTERS

Within the descriptions being compiled via the taxonomy in the MDG, there are specific rules defined to ensure data consistency. Although the MDG will apply these automatically when compiling the various descriptions, these rules need to be applied when entering the values in the MDG.

| SPECIAL CHARACTERS | RULES FOR SPECIAL CHARACTERS |
| --- | --- |
| . | Full stops will be used to break decimals. No spaces are allowed on either side of a full stop. No full stop are to be at the end of description. |
| / | A Forward (front) slash will only be used in Part numbers; Drawing numbers; Reference numbers or Fractions. No spaces are allowed on either side of a Front Slash. It is also used to separate values within a property (Used instead of the word “or”). I.e. 110 or 220 VAC = 110/220 VAC |
| - | A hyphen (dash) is allowed in the use of Part Numbers, Drawing numbers; Reference numbers or where grammatically necessary in descriptions. It is also used to indicate a Negative (minus) value. It is further used to indicate a range of values. (Used instead of the word “to”).I.e 110 to 220VAC = 110-220VAC No spacing allowed on either side. |
| x | An alone standing X is only allowed to denote "times" in sizes and will be in UPPER case. I.e. 20 X 50 X 10mm. If an X is part of the keyword or brand it MUST be UPPERCASE. |
| & | Ampersand may be used to replace the word AND to save space |
| % | Percentage may NOTbe used to describe proportion. The abbreviation "PCT" is to be used instead. |
| + | A Plus is allowed and us typically used in Tolerance. |
| +/- | A Plus Minus is allowed and is typically used in Tolerance. Do not use “±”. Note that ASCII code characters are not to be used. |
| ≥ | The "Greater than or Equal to" symbol is to be captured as ">=" Note that ASCII code characters are not to be used. |
| ≤ | The "Less than or Equal to" symbol is to be captured as "<=" Note that ASCII code characters are not to be used. |
| ° | The Degree sign is to be captured as DEG Note that ASCII code characters are not to be used. |
| “ | This is commonly used in industry to denote INCHES. This is not to be used. The INCH abbreviation “IN” is to be used instead. |
| ‘ | This is commonly used in industry to denote FEET/FOOT. This is not to be used. The FEET/FOOT abbreviation “FT” is to be used instead. |
| * | This symbol is often used to separate different dimensions / values in industry. This symbol is NOT to be used. The “X” is to be used. Eg. 150*100 is to be captured as 150 X 100. |
| Ø | This is very often used to denote “Diameter” or “Bore” and is not to be used. Use the prefix “DIA” instead. Note that ASCII code characters are not to be used. |
| MM² / MM³ | The miniature “²” or “³” are not to be used. MM2 or MM3 is the correct manner of capturing these values. Note that ASCII code characters are not to be used. |
| (3) DIA 10 MM HOLES | When there is more than a single quantity of a given value, the quantity is to be in brackets followed by the value. I.e. (3) DIA 10 MM HOLES means that there are 3 holes and the holes are 10 MM in diameter. |

### TOLERANCES

When Capturing Tolerances the following format is to be used.

Plus: the “+” shall be used, e.g. +0.001

Minus: the “-” shall be used, e.g. -0.0001

Plus/minus: “+/-“ shall be used, e.g.+/-0.0001

### SERIES OF NO’S

Apply the following rules for where values are capture in series. This is normally used for Bits, Sockets, Kits, etc.

| Single | 1,3,9,15,18 |
| --- | --- |
| Range | 1-3 -10 TO 15 |
| Single and Range combination | 1,3,7-9,15,18,20-25 |

### RANGES

Please apply the following rules when capturing ranges in value fields.

Note that when a range is from a negative to a positive then the word ”to” is used.

| Negative to Positive | -150 TO 120 |
| --- | --- |
| Positive to Positive | 120-150 |
| Minimum to Maximum | 120-150 |

### COLOR

When capturing colour(s) of items it is normally done alphabetically. E.g. Blue, Green Dark, Green Light, Yellow.

Colors are always captured with the Color name first followed by the shade. E.g. Green Dark

NOTE: This rule is NOT APPLIED to items where the color code defines ratings. E.g. Resistors, Pipes, Gaskets etc.Thesecolors sequencesmust not be changed and must be captured as per the Manufacturer specifications.

### DIMENSIONS, SIZES

Prefixes are added to Descriptions. E.g. ID 25XOD 35 MMXLG 6 M

Standard Abbreviations

| ABBREVIATION - EN | ABBREVIATION - ES | ABBREVIATION - FR | DESCRIPTION |
| --- | --- | --- | --- |
| DIA | DIA | DIA | DIAMETER |
| DP | PROF | PROF | DEPTH |
| HT | LAT | HT | HEIGHT |
| ID | DIA INT | DIA | INSIDE DIAMETER |
| OD | DIE EXT | DE | OUTSIDE DIAMETER |
| LG | LRG | LG | LENGTH |
| THK | ESP | EP | THICKNESS |
| SQ | CUAD | CAR | SQUARE |
| WD | LRG | LRG | WIDTH |

Format examples: Note the sequence in which these dimensions have been captured.

| DIMENSION / SIZE FORMAT | EXAMPLE |
| --- | --- |
| DIA X HT X DP | DIA 25 X HT 15 X DP 50 MM |
| DIA X LG | DIA 1/2 X LG 10 IN |
| DIA X LG X THK | DIA 50 X LG 200 X THK10 MM |
| DIA X THK | DIA 10 X THK 1/2 IN |
| ID X OD | ID 3 X OD 4-3/4 IN |
| ID X OD X LG | ID 12-1/2 X OD 15 X LG 24 IN |
| ID X OD X LG X THK | ID 10 X OD 20 X LG 150 X THK 1.5 MM |
| ID X OD X THK | ID 3/4 X OD 1-1/8 X THK 1/4 IN |
| SQ X DP | SQ 50 X DP 200 MM |
| SQ X DP X THK | SQ 50 X DP 25 X THK 2 MM |
| SQ X LG | SQ 12 X LG 20 IN |
| SQ X HT | SQ 3/4 X HT 4-5/16 IN |
| SQ X THK | SQ 65 X THK 45 MM |
| WD X LG | WD 1-1/2 X LG 2-3/4 M |
| WD X LG X DP | WD 250 X LG 800 X DP 50 MM |
| WD X LG X HT | WD 3-1/2 X LG 12 X HT 2 IN |
| WD X LG X THK | WD 35 X LG 1200 X THK 4 MM |

### UNIT OF MEASURES/NOTES

(These are NOT BUOM, but rather value UOM's)

Container Type

| ABBREVIATION | DESCRIPTION |  |
| --- | --- | --- |
| BG |  | Bag |
| BT |  | Bottle |
| BX |  | Box |
| CS |  | Case |
| CT |  | Carton |
| DR |  | Drum |
| DZ |  | Dozen |
| EA |  | Each |
| GR |  | Gross |
| JR |  | Jar |
| PD |  | Pad |
| PK |  | Pack |
| PL |  | Pallet |
| PR |  | Pair |
| TB |  | Tube |

Digital

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| GB | Gigabyte |
| KB | Kilobyte |
| MB | Megabyte |
| TB | Terabyte |

- Electrical Units/Ratings

When capturing electrical values the general order is as follows.  (This is used when there is no Value field to be able to capture values per characteristic).

If you have 2 different values for the same type of Voltage/Amp etc. always capture the Lowest First

Eg.: 120/240VAC

Volts, Amps, Phase, Watts, VA Rating, followed by the rest (Horsepower, Ohm, etc.), Hertz is always last.

I.e: 120/240VAC, 5A, 2PH, 50W, 3KV, 3HP, 5 OHM, 50/60HZ

There are, however items where the Electrical rating, should be split into separate Properties, as below:

- CURRENT = Ampere (A)

- POTENTIAL = Voltage (V)

- POWER = Wattage (W)

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| A | Ampere |
| AH | Ampere hour |
| AS | Ampere second |
| AC/DC | Alternating current/direct current |
| C | Coulomb |
| F | Farad |
| H | Henry |
| KA | Kilo Ampere |
| KV | Kilovolt |
| KVA | Kilo volt ampere |
| KW | Kilowatt |
| KWH | Kilowatt-hour |
| OHM | Ohm |
| VA | Volt-ampere |
| W | Watt |
| KW | Kilo Watt |

- Voltage units

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| V | Volt |
| VAC | Volt Alternative Current |
| VDC | Volt Direct Current |

- Frequency

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| GHZ | Gigahertz |
| HZ | Hertz |
| KHZ | Kilohertz |
| MHZ | Megahertz |

- Length

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CM | Centimetre (Only to be used for Clothing related dimensions) |
| FT | Feet |
| IN | Inch |
| KM | Kilometre |
| LG | Length |
| M | Meter |
| MI | Mile |
| MM | Millimetre |
| RL | Roll |
| YD | Yard |
| UM | Micron |

Note:

Less than 1000 MM = MM

Greater than 1000 MM = M

Greater than 1000 M = KM

- Nominal Pipe Sizes

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| SCH | Schedule |
| NPS | Nominal Pipe Size |

- Power Unit

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| HP | Horsepower |
| W | Watt |

- Pressure Ratings

DO NOT Convert PSIG, PSID or PSIA Ratings. 
DO NOT Convert PSI to LB for Pressure Gauges, Switches and Transmitters.
DO NOT Convert PSI to PSIG

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| BAR | Bar |
| HPA | Hecto Pascal |
| KPA | Kilo Pascal |
| MPA | Mega Pascal |
| MBAR | Millibar |
| PA | Pascal |
| PSI | Pounds per Square Inch |
| PSID | Pounds per Square Inch Differential |
| PSIG | Pounds per Square Gauge |

- Shape

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| RND | Round |
| SQ | Square |
| TRIA | Triangle |
| RECT | Rectangle |
| HEX | Hexagon/Hexagonal |
| OCT | Octagon |
| HEP | Heptagon |

- Speed

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CPM | Counts per minute |
| CW | Continuous wave |
| FT/MIN | Foot per minute |
| FT/SEC | Foot per second |
| KM/H | Kilometre per hour |
| MPH | Mile per hour |
| M/S | Meter per second |
| RPM | Revolutions per minute |
| RPS | Revolutions per second |

- Time

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| D | Day |
| HR | Hour |
| MIN | Minutes |
| MS | Millisecond |
| S | Second |
| YR | Year |
| MNTH | Month |

- Temperature Unit

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| DEG C | Degree Celsius |
| DEG F | Degree Fahrenheit |
| K | Kelvin |

- Volume

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CL | Centilitre |
| DL | Decilitre |
| DM3 | Cubic decimetre |
| DR | Drum |
| FL OZ | Fluid ounce |
| FT3 | Cubic foot |
| GAL | Gallon |
| L | Litre |
| M3 | Cubic meter |
| ML | Millilitre |
| MM3 | Cubic millimetre |

- Weight

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| CG | Centigram |
| DG | Decigram |
| G | Gram |
| HG | Hectogram |
| KG | Kilogram |
| LBS | Pound |
| OZ | Ounce |
| T | Metric ton (UK) |
| TON | Short ton (US) |
| TES | Tesla |
| MG | Milligram |
| ST | Stone |

### THREAD ABBREVIATION and NOTES

Note: If there is a sequence to capture for the thread information Capture it in a string format.
Capture as Thread Direction, Series, Class, Thread Size, Thread type.

Imperial Example

A typical example and breakdown of how thread data may be found is: 1/4-20UNC-2A,

- LH = if Left Hand (No symbol required for right hand thread).

- UNC = Thread series (Unified Coarse thread).

- 2A = Thread class symbol (A = External Tread and B = Internal Thread)

- 20 = Number of Threads Per Inch (TPI).

- 1/4 = Nominal Size.

Short Description. ex. LH,UNC,2A,20,1/4 IN
Long Description ex. LH,UNC,Cl 2A,20 TPI,1/4 IN

Metric Example

A typical example and breakdown of how thread data may be found is: M10, 1.25mm BSF, 2g

- LH = if Left Hand (No symbol required for right hand thread).

- BSF = Thread Type (British Standard Fine)

- 1.25mm = Thread Pitch

- 2g = Thread Class (Capital Letter = Internal, Small Letter = External Thread)

- M10 = Nominal size

Short Description. ex. LH,BSF,2G,1.25 MM,M10
Long Description ex. LH,BSF,CL2G,PITCH 1.25 MM,M10

Thread Class Imperial: 
A = External

B = Internal

Thread Class metric:

G/H = Internal

E/F = External

Thread Direction: 
RH = Right Hand (this is normally not captured as this is normally the default direction)
LH = Left Hand

| THREAD SERIES LIST |  |
| --- | --- |
| ABBRREVISION | DESCRIPTIONS |
| BSF | British Standard Fine |
| BSP | British Standard Pipe |
| BSPF | British Standard Pipe Fitting |
| BSPP | British Standard Pipe Parallel |
| BSPT | British Standard Pipe Taper |
| BSW | British Standard Whitworth |
| FBSP | Female British Standard Pipe |
| FNPT | Female National Pipe Taper |
| FPT | Female Pipe Taper |
| MPT | Male Pipe Taper |
| NPS | National Pipe Straight |
| NPT | National Pipe Taper |
| NST | National Standard Thread |
| UNC | Unified National Coarse |
| UNEF | Unified National Extra Fine |
| UNF | Unified National Fine |
| UNR | Unified National Round (Round Root) |
| UNS | Unified National Special |

### MATERIAL ABBREVIATION

| ABBREVIATION | DESCRIPTION |  |
| --- | --- | --- |
| AG |  | Silver |  |
| AL |  | Aluminum |  |
| ALALOY |  | Aluminium Alloy |  |
| APL |  | AluminiumPolyethylene Laminate |  |
| AS |  | Alloy steel |  |
| SAN |  | Styrene-Acrylonitrile (plastic) |  |
| ASB |  | Asbestos |  |
| AU |  | Gold |  |
| BMS |  | Bright mild steel |  |
| BRS |  | Brass |  |
| BRS CR PLTD |  | Brass Chrome Plated |  |
| BRS FRGD |  | Brass Forged |  |
| BRS NIPLTD |  | Brass Nickel Plated |  |
| BRS PLTD |  | Brass Plated |  |
| BRS SNPLTD |  | Brass Tin Plated |  |
| BRZ |  | Bronze |  |
| C |  | Carbon |  |
| CA |  | Cellulose Acetate |  |
| CANV |  | Canvas |  |
| CD |  | Cadmium |  |
| CHN STL |  | Chain Steel |  |
| CI |  | Cast iron |  |
| CI HRD NI |  | Cast Iron Hard Nickel |  |
| CEMNT |  | Cement |  |
| COMPD |  | Compound |  |
| CNCRT |  | Concrete |  |
| CPE |  | Chlorinated Polyethylene |  |
| CPVC |  | Chlorinated Polyvinyl chloride |  |
| CR |  | Chrome/chromium |  |
| CRMO |  | Moly chrome/chrome moly |  |
| CRSTL |  | Chrome steel |  |
| CS |  | Carbon steel |  |
| CS CAD |  | Carbon Steel Cadmium |  |
| CS CADPLTD |  | Carbon Steel Cadmium Plated |  |
| CS CTD |  | Carbon Steel Coated |  |
| CS CUPLTD |  | Carbon Steel Copper Plated |  |
| CS GALV |  | Carbon Steel Galvanized |  |
| CS PLTD |  | Carbon Steel Plated |  |
| CS RUBRLND |  | Carbon Steel Rubber Lined |  |
| CS GALV |  | Carbon Steel, Galvanized |  |
| CS PP LND |  | Carbon Steel Polypropylene Lined |  |
| CS ZNCTD |  | Carbon Steel Zinc Coated |  |
| CS ZNPLTD |  | Carbon Steel Zinc Plated |  |
| SS CST |  | Cast Stainless steel |  |
| CU |  | Copper |  |
| CU ALOY |  | Copper Alloy |  |
| CUALOYCST |  | Copper Alloy Cast |  |
| CU NI ALOY |  | Copper Nickel Alloy |  |
| CU NIPLTD |  | Copper Nickel Plated |  |
| CUSNPLTD |  | Copper Tin Plated |  |
| CVS |  | Chrome Vanadium Steel |  |
| DI |  | Ductile iron |  |
| EPTFE |  | Expanded Polytetrafluoroethylene |  |
| EP |  | Ethylene Propylene |  |
| EPDM |  | Ethylene Propylene Diene Monomer |  |
| EPM |  | Ethylene PropyleneMonomer |  |
| EPR |  | Ethylene Propylene Rubber |  |
| ETFE |  | Ethylene-tetrafluoroethylene |  |
| EVA |  | Ethylene Vinyl Acetate |  |
| FBR |  | Fibre |  |
| FBRGLS |  | Fiberglass |  |
| FE |  | Iron |  |
| FE GALV |  | Iron Galvanized |  |
| FE NIPLTD |  | Iron Nickel Plated |  |
| FE OXID |  | Iron Oxide |  |
| FEP |  | Fluorinated Ethylene Propylene |  |
| FLPE |  | Fluorinated Polyethylene |  |
| GALV |  | Galvanized |  |
| GALVCTD |  | Galvanized Coated |  |
| GE |  | Germanium |  |
| GI |  | Galvanised iron |  |
| GLS SFTY |  | Glass Safety |  |
| GMS |  | Galvanised Mild Steel |  |
| GRP |  | Glass Reinforced Plastic |  |
| GS |  | Galvanised steel |  |
| HCS |  | High Carbon Steel |  |
| HDPE |  | High Density Polyethylene |  |
| HDPP |  | High Density Polypropylene |  |
| HDPU |  | High Density Polyurethane |  |
| HG |  | Mercury |  |
| HNBR |  | Hydrogenated Acrolyontrile Butadiene Rubber |  |
| HRS |  | Hot Rolled Steel |  |
| HS |  | Hassium |  |
| HSS |  | High Speed Steel |  |
| HTS |  | High Tensile Steel |  |
| LCP |  | Liquid Crystal Polymer |  |
| LDPE |  | Low Density Polyethylene |  |
| LTHR |  | Leather |  |
| LTS |  | Low Tensile Steel |  |
| MDPE |  | Medium Density Polyethylene |  |
| MG |  | Magnesium |  |
| MGO |  | Magnesium Oxide |  |
| MI |  | Malleable Iron |  |
| MN |  | Manganese |  |
| MO |  | Molybdenum |  |
| MPTFE |  | MultifullPolytetrafluoroethylene |  |
| MS |  | Mild Steel |  |
| MS EPXYCTD |  | Mild Steel Epoxy Coated |  |
| MS GALV |  | Mild Steel Galvanized |  |
| NBR |  | Nitrile Butadiene Rubber |  |
| NI |  | Nickle |  |
| NIHRD |  | Nickel Hard |  |
| NIPLTD |  | Nickel Plated |  |
| NIPLTD CU |  | Nickel Plated Copper |  |
| NI STL |  | Nickel Steel |  |
| NPR |  | Neoprene |  |
| NYL |  | Nylon |  |
| P |  | Phosphorus |  |
| PA |  | Polyamide |  |
| PAN |  | Polyacrylonitrile |  |
| PB |  | Lead |  |
| PC |  | Polycarbonate |  |
| PE |  | Polyethylene |  |
| PET |  | Polyethylene Terephthalate |  |
| PEVA |  | Polyethylene Vinyl Acetate |  |
| PI |  | Polyimide |  |
| PLSTC |  | Plastic |  |
| PLSTC FBR |  | Plastic Fibre |  |
| PLSTC HRD |  | Plastic Hard |  |
| PLTD BRS |  | Plated Brass |  |
| PLSTC FE |  | Plated Iron |  |
| PLSTC STL |  | Plated Steel |  |
| PLYMR |  | Polymers |  |
| PLYF |  | Polyolefin |  |
| PP |  | Polypropylene |  |
| PPO |  | Polypropylene Oxide |  |
| PPS |  | Polypropylene Sulphide |  |
| PT |  | Platinum |  |
| PTFE |  | Polytetrafluoroethylene |  |
| PU |  | Polyurethane |  |
| PVA |  | Polyvinyl Acetate |  |
| PVAL |  | Polyvinyl Alcohol |  |
| PVB |  | Polyvinyl Butral |  |
| PVC |  | Polyvinyl Chloride |  |
| PVCA |  | Polyvinyl chloride acetate |  |
| PVDC |  | Polyvinyl Dene chloride |  |
| PVDF |  | Polyvinyl Dene Fluoride/Difluoride |  |
| PVF |  | Polyvinyl Fluoride |  |
| PVFM |  | Polyvinyl Formal |  |
| PVS |  | Polyvinyl Siloxane |  |
| RH |  | Rhodium |  |
| RPTFE |  | Reinforce Polytetrafluoroethylene |  |
| SBR |  | Styrene Butadiene Rubber |  |
| SGI |  | Spheroidal Graphite Iron |  |
| SHT MTL |  | Sheet Metal |  |
| SHT STL |  | Sheet Steel |  |
| SI |  | Silicone/Silicon |  |
| SIC |  | Silicon Carbide |  |
| SN |  | Tin |  |
| SS |  | Stainless Steel |  |
| STL |  | Steel |  |
| TI |  | Titanium |  |
| TPE |  | Tetrafluroethylene |  |
| TPE |  | Thermoplastic Elastomer |  |
| UPES |  | Unsaturated Polyester |  |
| UPVC |  | Un-practiced polyvinyl chloride |  |
| V |  | Vanadium |  |
| WC |  | Tungsten Carbide |  |
| WI |  | Wrought Iron |  |
| WL |  | Wool |  |
| WS |  | Wrought Steel |  |
| ZN |  | Zinc |  |
| ZR |  | Zirconium |  |

### GENERAL ABBREVIATION

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| AWS | American Welding Association |
| AWG | American Wire Gauge |
| BC | Bayonet Cap (lamp) |
| BTU | British thermal unit |
| CD | Cadela (luminous Intensity) |
| CAL | Calorie |
| CARAT | Carat |
| CA | Centiare |
| Cl | Class |
| CES | Candelabra Edison Screw (lamp) |
| C/W | Come With |
| C | Core (cable) |
| CSK | Countersunk |
| DB | Decibel |
| DEG | Degree / ° |
| DN | Dimension Nominal |
| DP | Double Pole |
| DPDT | Double Pole , Double Throw |
| DPST | Double Pole , Single Throw |
| DWG | Drawing |
| DUPL | Duplicate |
| ELECMAG | Electromagnetic |
| ES | Edison Screw (lamp) |
| F | Farad |
| FF | Flat Face |
| FL OZ | Fluid ounce |
| FT LBS | FootPound |
| FR | Front |
| HD | Head |
| HRS | Hours |
| HYDR | Hydraulic |
| G | Gauss |
| GR | Grade (material) |
| GES | Giant Edison Screw (lamp) |
| HEX | Hexagon/Hexagonal |
| HPS | High Pressure Sodium |
| IMP | Impulse |
| J | Joule |
| KJ | Kilojoule |
| LH | Left Hand |
| LBS | Pound |
| LM | Lumen |
| LES | Lilliput Edison Screw (lamp) |
| LDR | Light Dependent (Resistor) |
| LUX | Lux |
| M | Male |
| MNFR | Manufacturer |
| MAX | Maximum |
| MFD | Microfarad |
| MBC | Miniature Bayonet Cap |
| MES | Miniature Edison Screw (lamp) |
| MW HR | Megawatt hour |
| UM | Micron / μm |
| MIN | Minimum |
| MISC | Miscellaneous |
| N | Newton (force) |
| NB | Nominal Bore |
| N/M2 | Newton per meter squared |
| N/S | Newtons per second (rate of force development) |
| NC | Normally Closed |
| NO | Normally Open |
| OEM | Original Equipment Manufacturer |
| PPM | Part per million |
| PCT | Percent |
| PERM | Permanent |
| PCS | Piece |
| PLTD | Plated |
| P | Pole |
| PH | Phase |
| PM | Post Meridiem |
| PDR | Potential Duplicate Report |
| PSIA | Pounds per square inch absolute |
| PRES | Pressure |
| RAD | Radian |
| REAR | Rear |
| RF | Raised Face |
| RH | Right Hand |
| SBC | Small Bayonet Cap (lamp) |
| SCKT | Socket |
| SES | Small Edison Screw (lamp) |
| SFTWR | Software |
| SSR | Solid State (Relay) |
| SUPL | Supplier |
| TEMP | Temperature |
| THD | Thread/Threaded |
| VSD | Variable Speed Drive |
| WH | Watt-hour |
| YEL | Yellow |

### Examples per material (item) types

This section is to supply information for a particular material master type. Please note that not all material types are in examples below.

Material is to be captured as Material and Material Grade E.g. Stainless Steel 316L, further the Material Specification must be captured where available E.g ASTM A193 B8 CL1

### Metric to Imperial Conversion

All imperial sizes are to be fractionalized.

E.g. 1.5 IN = 1-1/2 IN

Here is a good website to use to do conversions –

https://www.metric-conversions.org/length/inches-to-millimeters.htm

Select “Inches to “Millimetre’s”

Imperial size converted to metric in millimetres.

Enter imperial decimal size.

Highlight and copy.

Paste copied millimetre’s value

Click on “Swap Units”

Remove “mm” at the end of number

Imperial fraction is given and to be used.